# 1.  Product Description

This manual contains the information you will need to install and use the EXM-2 Flash Disk and accompanying software.

EXM-2 is a semiconductor disk device implemented using flash memory technology.  It is designed to plug into any slot in an EXMbus carrier.  A variety of EXM-2's are available with different memory capacities.  EXM-2 appears to software as a write-protected disk, and it may serve as a boot disk (e.g., for diskless operation).  A program is provided to format EXM-2 as a disk and copy files to it.

Although attempts to write to EXM-2 through the operating system and/or disk BIOS will fail with a write-protect error, there is an EXM-2 specific way to erase and write the flash memory devices.  For users needing the extra security of ensuring that programs cannot do this, a write-protect jumper is provided.

The formatting program is provided on a floppy diskette included with the EXM-2.  You should read the readme.txt file on the floppy diskette for update information.

If you are unsure of the capacity of your EXM-2, it can be determined by counting chips.  If it contains flash chips ending in "010", there are eight chips per megabyte.  If it contains flash chips ending in "020", there are four chips per megabyte.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-2.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 125° C |
| Humidity | operating | 0 - 95% (non-condensing) |
|  | storage | 0 - 95% (non-condensing) |
| Vibration | operating | .015"displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | .030" displacement with 5 g peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30g 11 msec duration, half-sine shock pulse |
|  | storage | 50g 11 msec duration, half-sine shock pulse |
| Electrical |  |  |
| Current | maximum | 5V @ 500 mA |
|  | maximum | 12V @ 50 mA |

Table 1. EXM-2 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.