# 2.  Installation

Before installing the EXM-2, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-2, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Write-Protection Jumper

When you install your EXM-2, ensure that the write-protection jumper is installed in the write-enabled position.  As shown in the diagram below, the jumper should connect the left and middle pins.

Figure 1. EXM-2 Jumpers.

## Insertion in an EXM Carrier

Once the header pins are set, insertion of the EXM-2 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-2 into the card guides.  Firmly press the 
EXM-2 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Running the EXM Configuration Program

For the EXM-2, the configuration program simply specifies in which slot the 
EXM-2 resides, and whether the EXM-2 is enabled.  This information is recorded in the EPC's nonvolatile memory and is used by the EPC's BIOS at initialization time.

To use the configuration program, you can execute it from the floppy diskette or first copy it from the floppy diskette to a hard disk.  For instance, to execute it from the floppy diskette, put the diskette in your EPC and type a:exmcfg

The configuration program must be executed on the EPC in which you've installed the EXM-2.  If you cannot execute the program, there is a lower-level way to configure the EXM-2 via the EPC's BIOS setup screen.  Refer to the next section.

The configuration program displays a screen showing the devices installed in each EXMbus slot.  Follow the instructions and select the slot in which your EXM-2 is installed. The next screen, specific to the EXM-2, asks you about the only configuration option available - whether you want the EXM-2 enabled or not.  If you have only one EXM-2 in the system, you should answer yes (enabled).  If you have multiple EXM-2 modules in the same system, you must not enable more than one of them (enabling and disabling can be done dynamically by software).

Follow the remainder of the instructions on the screen of the configuration program to save the specified configuration in the EPC's nonvolatile RAM.  After this, every time you boot or reset your EPC, the BIOS will initialize the EXM-2 in the specified slot to the specified configuration.

## Configuring the BIOS Setup

Once the EXM-2 is inserted into the carrier, turn the system on and boot it up. The EXM configuration data in the EPC to which the EXM-2 is connected now must be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function from the command line by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    DC  00  00
      1    DB  C1  00
      2    FD  01  00
      3    DE  00  39
      4    F5  05  00
      5    DC  F5  00

Figure 2. EXM Setup Screen.

The EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  An example of configuration data for the EXM-2 is shown in bold in slot 2.  SLOT, ID, OB1 and OB2 are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-2 occupies.  Dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 3. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

Note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module show an ID of FF and OB1/OB2 of   00  00  indicating  no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-2 should be set to FDh.

OB1 is a hexadecimal value derived by combining the following:

| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 2 | Bit 1 Card Enable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 disable 1 enable |

Figure 4. OB1 Register.

A typical OB1 value is 01h (00000001) indicating the card is enabled.

OB2 is not used and should be set to 00.