3.  Programming Interface

This chapter contains information needed to write custom software drivers for the EXM-2.  Anyone just using the supplied software that supports EXM-2 as a disk device can skip reading this chapter.

The EXM-2 defines the following registers in the I/O space.

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Reg | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg |  |  |  |  |  |  |  | Cden | 102 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Low Address Register | Address Bits 7 - 0 | 8380 |
| --- | --- | --- |

| Middle Address Register | Address Bits 15 - 8 | 8381 |
| --- | --- | --- |

| High Address Register | VPPEN Address Bits 22 - 16 | 8382 |
| --- | --- | --- |

| Data Register | Data Bits 7 - 0 | 8383 |
| --- | --- | --- |

The first two are standard EXM registers for device identification and configuration.  The EXM-2  responds to accesses to ports 100h and 102h only if its EXMbus line 
-EXMID is asserted.  An 8-bit read from I/O address 100h returns the value FD, the device ID of the EXM-2.  A read/write configuration register appears at I/O address 102h.  The only defined bit is CDEN, which specifies whether the EXM-2 is enabled (1) or disabled (0).  If disabled, the EXM-2 will not respond to the 8380-8383 I/O addresses; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h, and then only if EXMbus line -EXMID is asserted.

The flash devices are accessed by placing an address in the three address registers and then reading or writing the data register, which causes the data byte at the location specified in the address registers to be read or written.  An EXM-2 can contain up to 24 1 Mb or 2 Mb flash memory devices.  The high-order five bits (18-22) select the device and the remaining 18 bits drive the address lines of that device.  The three address registers are write-only.

The VPPEN bit enables the Vpp erasing and programming voltage to the flash devices through a timer.  To erase or program, one must set VPPEN before starting the programming or erasing process, and then continuously toggle VPPEN (alternately change to 0 and 1) every 70 milliseconds or sooner during the programming or erasing process.  The rationale for this is that flash devices destroy themselves if left in the programming or erasing state for an excessive period of time (which may occur, for instance, if one reset the EPC system during this state).  The timer helps prevent this by removing the Vpp voltage unless VPPEN is being continually accessed.

The details of manipulating the flash devices, such as using their command register and identifier, are not specified here; consult the datasheets for the Intel 28F0x0 flash memory devices.  Note that some functions, such as reading the device IDs, require the write-protection jumper to be in the write-enabled position.

The details of the data structures built in the flash devices by the flashfmt program are not specified here but are available upon request.

## Formatting Program

The first thing that you are likely to do with the EXM-2 is format it and create directories and files.  This is performed by a program called flashfmt.  It is distributed on the floppy diskette shipped with EXM-2.

Flashfmt operates on the currently enabled EXM-2, whose write-protection jumper must be set as write-enabled.  It erases the flash memory, formats it (with FATs, cylinders, heads, and sectors as if it were a disk), optionally adds files to make it a system boot device, and optionally copies a directory structure and files into it.  The program is invoked with the following command line

flashfmt    [options]    directory

All files in directory directory (typically a directory on your hard disk) and all of its subdirectories (if any) and their files are copied into the EXM-2.  Each file is created with the same permissions as the corresponding source file, except that the files are marked as read-only and unarchived (see the DOS ATTRIB command).

The options are

/S	Make a system (bootable) disk.  The two DOS hidden files and the COMMAND.COM file are copied into the outermost directory on EXM-2.  These files come from the current boot device of the EPC.

/B=x	Make a system disk, taking the system files from a specific device (e.g., A:) or directory (e.g., \DOSDIR) specified as x.

/V	Prompt for a volume label (in the same manner as the DOS format command).

Some examples are described below.

flashfmt /b=a: a:

Formats the EXM-2 as a DOS system disk from the DOS system disk in drive a:, and copies all files from device a: into the EXM-2 (including subdirectories).

flashfmt /s c:\pmon

Formats the EXM-2 as a DOS system disk and copies all files and subdirectories in and beneath directory pmon into the EXM-2.

flashfmt a:

Formats the EXM-2 and copies all files from device a: into the EXM-2 (including subdirectories).

Flashfmt automatically determines the amount of available space on your EXM-2.  It also calculates the amount of space needed for the files to be copied, and aborts without erasing the EXM-2 if there is insufficient space.

Flashfmt aborts if the source device (for either the file copy or for the system files) is the same EXM-2 as being formatted.

Since the EXM-2 is represented to DOS as a fixed (nonremovable) disk, DOS may have buffered data from the EXM-2's files prior to its being formatted.  As a result, you should reboot your system prior to accessing files in a newly formatted EXM-2.