#include <iostream.h>

#include <memory.h>

#include <conio.h>

typedef unsigned char UCHAR;

typedef unsigned short USHORT;

typedef unsigned long ULONG;

#define FAR		_far

#define BYTESPERSECTION 0x200

#define EXMID		0x96

#define EXMENABLE	0x00000001

#define LSWLOWBYTE	0x8380

#define LSWHIGHBYTE	0x8381

#define MSWLOWBYTE	0x8382

#define MSWHIGHBYTE	0x8386

#define SRAMDATA	0x8384

UCHAR	Slot	=	5;		/* presumes EXM-2 in slot 5 */

void

readsection(UCHAR FAR *target, ULONG sramoffset)

{   UCHAR ob;

register USHORT rinductor;

/*

//	Enable the card to allow register access

*/

outp(EXMID,Slot);

ob = inp(0x102);

outp(0x102,ob | EXMENABLE);

/*

//	Load initial offset value (sramoffset should be divisible by 256)

*/

outp(MSWHIGHBYTE,(USHORT) (sramoffset >> 24));

outp(MSWLOWBYTE,(USHORT) (sramoffset >> 16));

outp(LSWHIGHBYTE,(USHORT) (sramoffset >>8));

outp(LSWLOWBYTE,(USHORT) sramoffset);

/*

//	Read a section using the autoincrement feature of the EXM-2A.

*/

for (rinductor = 0; rinductor < BYTESPERSECTION; rinductor++) {

*target++ = (UCHAR) inp(SRAMDATA);

}

outp(0x102,ob);	/* restore the option byte */

}

void

writesection(UCHAR FAR *source, ULONG sramoffset)

{   UCHAR ob;

register USHORT rinductor;

/*

//	Enable the card to allow register access

*/

outp(EXMID,Slot);

ob = inp(0x102);

outp(0x102,ob | EXMENABLE);

/*

//	Load initial offset value (sramoffset should be divisible by 256)

*/

outp(MSWHIGHBYTE,(USHORT) (sramoffset >> 24));

outp(MSWLOWBYTE,(USHORT) (sramoffset >> 16));

outp(LSWHIGHBYTE,(USHORT) (sramoffset >>8));

outp(LSWLOWBYTE,(USHORT) sramoffset);

/*

//	Write a section using the autoincrement feature of the EXM-2A.

*/

for (rinductor = 0; rinductor < BYTESPERSECTION; rinductor++) {

outp(SRAMDATA,*source++);

}

outp(0x102,ob);	/* restore the option byte */

}

void

main()

{   UCHAR test[BYTESPERSECTION];

UCHAR readbuffer[BYTESPERSECTION];

register USHORT minductor;

for (minductor = 0; minductor < BYTESPERSECTION; minductor++)

test[minductor] = minductor;

writesection(test,0);

readsection(readbuffer,0);

if (memcmp(test,readbuffer,BYTESPERSECTION))

cout << "Read/Write test failure" << endl;

else cout << "Read/Write test successful" << endl;

}