# 1.  Product Description

This manual contains the information you will need to install and use the EXM-2A expansion module and accompanying software.

The EXM-2A is a semiconductor-based disk device implemented using flash and SRAM memory technology.  A variety of different configurations and memory capacities are available.

The flash memory portion of the EXM-2A appears to software as a read-only hard disk, and it may serve as a boot disk (e.g., for diskless operation).  The SRAM portion of the EXM-2A is read/write and appears as a non-bootable disk. Software is provided to format the SRAM and flash portions of the EXM-2A.

A read-only jumper for flash memory is provided.

A formatting program and an SRAM driver are provided on a floppy diskette included with the EXM-2A.  Read the file readme.txt found on the EXM-2A distribution diskette for any updated information not included in this manual.

µ §µ §
If you are unsure of the capacity of your EXM-2A, capacity can be determined by reading the part number sticker placed on the module itself (see Figure 1). In this example, a card with the part number EXM2A-02-512 has two megabytes of flash memory and 512K of SRAM.

Figure 1. EXM-2A ID Sticker.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-2A.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 85° C |
| Humidity | operating | 5% - 95% (non-condensing) |
|  | storage | 5% - 95% (non-condensing) |
| Vibration | operating | .015"displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | .030" displacement with 5 g peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30g 11 msec duration, half-sine shock pulse |
|  | storage | 50g 11 msec duration, half-sine shock pulse |
| Electrical |  |  |
| Voltage | +5V Supply | +5V ± 5% |
|  | +12V Supply | +12V ± 5% |
| Current | typical | 5V @ 400 mA |
|  | typical | 12V @ 40 mA |
| Miscellaneous |  |  |
| Weight |  | 3.0 ounces |

Table 1. EXM-2A Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.