# 2.  Installation

Before installing the EXM-2A, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-2a, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Before You Begin

The process for installing and formatting the EXM-2A is as follows:

1. 	Set the read-only jumper, if necessary (see page 17).

2.	Insert the EXM-2A into an EXM carrier.

3.	Configure the C: and D: drive(s) in the BIOS as required.

4.	Set the EXM-2A configuration in the EXM Setup Screen.

5.	Format the EXM-2A flash and SRAM as required.

6. 	Load the SRAM device driver.

## Read-Only Jumper

If you wish to protect the flash memory from being reprogrammed by software, make sure the JP1 jumper is in the read-only (or write-disabled) position. The factory default position for this jumper is read/write (write-enabled). Note that the EXM-2A cannot be formatted if the jumper is in the read-only position.

µ §

Figure 2. EXM-2A Read-Only Jumper.

## Insertion into an EXM Carrier

Insertion of the EXM-2A into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-2A into the card guides.  Firmly press the EXM-2A front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews. DO NOT OVERTIGHTEN.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

Please note that the EXM-2A will not fit into slot 1 (top EXM slot) of an EPC-4 or slot 1 of an EPC-5.  If slot 1 is the only open EXM slot of your EPC-4 or EPC-5, it will be necessary to move another EXM to slot 1 and install the EXM-2A in the vacated slot. Make sure to change all the appropriate values in the EXM Setup Screen.

## BIOS Setup Screen

The EPC BIOS contains a setup function to display and alter the system configuration.  This configuration information is maintained in the EPC's battery-backed CMOS RAMxe "CMOS RAM"§ and is used by the BIOS to initializeXE "initialization"§ the system hardware.

Once the EXM-2A is inserted into the carrier, turn the system on and boot it up.  The setup function can be invoked any time after the POST completes and first clears the screen.  Simultaneously press the CTRL+ALT+ESC keysxe "CTRL+ALT+ESC"§.  This may be done during system operation in most, but not all circumstances.  Some programs that take control of the keyboard at a low level, such as Microsoft Windowsxe "Windows"§, cause this key sequence to be interpreted differently, or not at all.  It should always work, however, when the DOS operating system prompt is shown on the screen.

The main setup screenxe "setup screen"§ displays similar to the following:

RadiSys EPC-23 CMOS Setup, BIOS V3.05

486SL, 4 MBytes memory

Date (mm/dd/yy) .............. 01/12/93

Time (hh:mm:ss) .............. 07:34:56

Configuration Errors.......... Halt on all errors

Diskette Drive A ............. 1.4M 3.5 inch

Diskette Drive B ............. none

Fixed Disk Drive C .... Flash

Fixed Disk Drive D .... none

...

...

Figure 3. EXM Main Setup Screen.

## Configuring the Fixed Disk

Only the relevant part of the EXM Main Setup Screen, which concerns the fixed disk drive, is shown in Figure 2. xe "Fixed disk drive"§This display-only field shows the type of disk selected from the fixed disk menu.  Possible values vary with different CPUs. Not all values listed here apply to each CPU. Also, note that "Flash" may appear as "EXM Flash" in some systems. Possible values are None, AT, SCSI, VME and FLASH.  To see the detailed characteristics of the device or to change the device, use the F3 function key to go to the Fixed Disk MenuXE "fixed disk menu"§.  xe "EXM configuration"§xe "EXM configuration program"§The Fixed Disk Menu is used to define the type of hard disk(s) installed in the system.

The Fixed Disk Menu Screen resembles the following:

RadiSys EPC-23 Fixed Disk Menuxe "Fixed Disk Menu"§, System BIOS V3.05
486SL, 4 MBytes memory,

Fixed Disk Drive C:  Flash
       EXM-2

Fixed Disk Drive D:  None
  
                                      	F10 = Save and return
 xe "VXI Register Base"§			xe "ULA"§ESC = Return without saving
                                       	 ¯  ¿ move between items

¬ ® select values

Figure 4. Fixed Disk Menu Screen.

Use the up and down cursor keys ( ¯) to move between items.  Use the left and right cursor keys (¬ ® ) to scroll through the available choices for each item.

Choose disk type Flash. An EXM flash disk can be made the boot device by making Flash the drive C: type.

Note that flash disks cannot be used as drive C: when a standard hard disk (SCSI or AT) is used as drive D:. Instead, set up the standard hard disk as drive C: and set up the flash disk as drive D:.

Once configuration of the fixed disk is complete, press the F10 key to return to the main setup screen.

## Enabling the EXM module

Once in the setup program, a menu displays which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    EC  01  00
      2    FD  01  00
      3    7C  E1  FF
      4    F5  05  00
      5    FA  25  00

Figure 5. EXM Setup Screen.

The EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  An example of configuration data for the EXM-2A is shown in bold in slot 2.  SLOT, ID, OB1 and OB2 are defined as follows:

SLOT	indicates the slot where the EXM is installed.  See the diagram below to determine which EXM slot the EXM-2A occupies.  Dashed lines indicate EXM slots not available on all systems.

EXM Slots

µ §

Figure 6. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

All slots not occupied by an EXM module show an ID of FF and OB1/OB2 of   00  00  indicating no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-2A should be set to FDh.

OB1 is a hexadecimal value derived by combining the following:

| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 Revision | Bit 0 Card Enable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x | x | x | x | x | x | 0 | 0 disable 1 enable |

Figure 7. OB1 Register.

Bits 7-2 are undefined.

Bit 1 is a "Revision" bit and is set to 0 to help software distinguish this card from an EXM-2. This bit is not affected by writes.

Bit 0 is the standard EXM Card Enable bit. When set to 0, the card is disabled. Set the bit to 1 to enable the EXM-2A.

A typical OB1 value is 01h (00000001) indicating the card is enabled.

OB2 is not used and should be set to 00.