# 3.  Operations

## Distribution Diskette Contents

The EXM-2A distribution diskette contains the following files:

XFORMAT.EXE	Flash formatting program

SRAMDISK.SYS	SRAM device driver

BB5.00			Boot block files for DOS 5.0

BB6.00			Boot block files for DOS 6.0, 6.1, & 6.2

README.TXT		Additional user information added after the 					manual was printed

## Installation

Create a directory on the C: drive called C:\EXM2A (or another name you choose). Copy the files from the distribution diskette to the C:\EXM2A directory.

## Formatting Program

The first task that you are likely to perform with the EXM-2A once it is installed and configured is format the flash memory and copy directories and files to it.  This procedure is performed by the program named XFORMAT.  It is distributed on the floppy diskette shipped with the EXM-2A. This program formats both flash and SRAM.

When xformat is used to format flash memory, the function first checks the status of the battery to determine if enough voltage is present, then erases all flash memory, formats it, optionally adds system files to make it a boot device, and the optionally copies a directory structure and files into it. The program is invoked with the following command line:

XFORMAT    [options]    [srcdir]

All files in directory srcdir (typically a directory on your hard disk) and all of its subdirectories (if any) and their files are copied into the EXM-2A.  To make changes to the flash disk you must rerun the XFORMAT function.

Because SRAM disks are read/write, the [srcdir] parameter is optional for SRAM drive types. Files can be written to an SRAM device using standard DOS copy commands (COPY, XCOPY).

For flash, each file is created with the same attributes except the files are also marked as read-only and unarchived (see the DOS ATTRIB command for more information).  For SRAM, each file is created with the same attributes as the corresponding source file.  The date/time stamp for file modification or creation stays the same also.

The following flags are available when using the XFORMAT function. Note that a dash (-) can be used interchangeably with a slash (/) to set flags.

/B=<sysdir>	Boot Disk Flag. Creates a bootable disk image using the DOS system files specified by the <sysdir> parameter. The function formats and copies files to the formatted disk. A bootable disk image is created. 	The operation fails if it cannot find the operating system files.

It is possible to create a bootable disk image from a non-bootable source drive. Refer to Appendix B for details.

This flag is for use with flash disks only.

/F<name> <size>

File Output Flag. Outputs the disk image to the file <name> with size <size>.  <size>  is a hex value that specifies the number of Kbytes in the target file. This option is useful for creating VME memory disks and images for users that are going to write their own flash writing program, and is not strictly applicable to only the EXM-2A, but is documented here for completeness.

/H		Help Flag. Displays a list of available flags and their options.

/N=<ver>	Non-Boot Version Flag. Creates a bootable disk image using the boot block file specified by the <ver> parameter.  (The location of the boot block file is specified by /B or /S flags.) This option is useful for creating bootable flash disks from non-bootable sources such as network and RAM disk drives (see Appendix B for more information on creating bootable images from non-bootable disks). The boot block file for /N=5 is BB5.00 (DOS 5) and the boot block file for /N=6 is BB6.00 (DOS 6.X). The boot block files are supplied with the EXM-2A.

This operation fails if it cannot find the operating system files.

This flag is for use with flash disks only.

/P=<slot>	Position Flag. The slot number specified is the EXM slot number where the EXM-2A is located in the chassis.

/Q=m 	Quiet Mode Flag. Suppresses progress display messages. May be combined with the No Reboot Flag (i.e., /Q=mr).

/Q=r	No Reboot Flag. Suppresses reboot upon successful completion. May be combined with the Quiet Mode Flag (i.e., /Q=rm).

It is necessary to flush the DOS buffer areas prior to accessing newly formatted disks. This is accomplished by rebooting. This reboot suppression flag is strictly for use when XFORMAT is used in a batch file where the reboot command is at the end of the batch file.

/S	System Flag. Creates a bootable disk image using the DOS system files on the disk that XFORMAT is invoked from.

This operation fails if it cannot find the operating system files.

This flag is for use with flash disks only.

/T=<type>	Type Flag. Specifies the type of format to be performed. When <type> is S, the function formats the SRAM. When <type> is O, the function formats the onboard flash (flash memory on the CPU board, not EXM-2A flash memory). When /T=O is selected, the /P flag is not allowed.

Specifying no option results in formatting the EXM flash.

/V 	Volume Label Flag. Prompts for a disk volume label. A volume label identifies the disk and can be a maximum of 11 characters. The label conforms to the MS-DOS convention, thus the following are not allowed:  * . + , : ; < > =  ? [ ] \ / | ( )  ^ & . Label letters are converted to upper case.

Note that not all flags are valid for every system. For example, /T=O is used only for systems that contain onboard flash memory (i.e., flash memory on the CPU). Also, the /S, /B, and /N flags are not valid for SRAM drives and generate an error message when attempts are made to use these flags on SRAM.

Some examples of the XFORMAT invocation are described below.

XFORMAT /B=a: a:

Formats the EXM-2A flash memory as a DOS system disk from the DOS system disk in drive a:, and copies all files from device a: into the EXM-2A (including subdirectories).

XFORMAT /T=S C:\SRAM

Formats the EXM-2A SRAM and copies all files and subdirectories of directory C:\SRAM into the EXM-2A.

XFORMAT a:

Formats the EXM-2A flash memory and copies all files from device A: into the EXM-2A (including subdirectories). The EXM-2A will not be bootable if not formatted with the /S or /B flag.

XFORMAT /N=6 /b=d: C:\FLASH

Format the flash memory using the system files and the boot block file BB6.00 from directory D:\ and copy all files from C:\FLASH. Note that you cannot simply copy a file to a flash disk. You must use XFORMAT to erase, format, and copy the contents of a directory to the flash disk.

Unless the /F option is selected, XformAT automatically determines the amount of available space on your EXM-2A.  It also calculates the amount of space needed for the files to be copied, and aborts without erasing the EXM-2A if there is insufficient space.

XFORMAT aborts if the source device (for either the file copy or for the system files) is the same EXM-2A being formatted.

Since the EXM-2A is represented to DOS as a fixed (non-removable) disk, DOS may have buffered data from the EXM-2A's files prior to its being formatted.  As a result, you should reboot your system prior to accessing files in a newly formatted EXM-2A.

## SRAM Disk Device Driver

The SRAM driver SRAMDISK.SYS is used to configure the EXM-2A SRAM as a disk. The SRAM disk is not bootable, and is not visible to the operating system until the SRAM is formatted using XFORMAT, and the driver is loaded.

When SRAMDISK.SYS is loaded, it will check the status of the battery voltage. If the battery voltage is above 2.5V, the driver will load. If the battery voltage is below 2.5V, an error message displays for a few seconds before continuing.

To load the SRAMDISK.SYS driver, edit the CONFIG.SYS file and insert the following line:

DEVICE=[path]SRAMDISK.SYS /[slot]

The SRAM driver can be loaded high to conserve conventional memory. Use the following line under DOS 6.X:

DEVICEHIGH=[path]SRAMDISK.SYS /[slot]

The path you insert depends on what directory you create when you copy the 
EXM-2A software from the RadiSys distribution diskette to your target drive.

The slot you select depends on where the EXM-2A is installed in the EXM Setup Screen. For example, to configure the SRAM in an EXM-2A located in slot 3, use the following command:

DEVICE=C:\EXM2A\SRAMDISK.SYS /3

When installing SRAM drives, make sure you run XFORMAT on each drive before running the SRAMDISK.SYS driver for that drive. The driver(s) will not recognize the SRAM disk until it has been formatted using XFORMAT.

SRAM data integrity may be jeopardized when power is removed during a series of write operations to the SRAM. For this reason, XFORMAT checksums each sector. The SRAMDISK.SYS driver must compute the same checksum in order for the sector read to be successful. Corrupt sectors may be repaired with standard disk tools such as Norton Utilities.

## Error Messages

The following alphabetically listed error messages display during error conditions using XFORMAT.EXE or SRAMDISK.SYS.

| Error Message | BATTERY NEEDS REPLACEMENT |
| --- | --- |
| Explanation | The XFORMAT program or the SRAMDISK.SYS driver have detected that the battery power is below 2.5V. |
| Resolution | Replace the battery. Refer to Appendix A. |
| Error Message | CANNOT READ COMMAND.COM |
| Explanation | The command processor COMMAND.COM could not be located. |
| Resolution | Copy COMMAND.COM to the appropriate directory and re-invoke XFORMAT. |
| Error Message | <path> COMMAND.COM WILL NOT BE COPIED TO THE TARGET |
| Explanation | A version of COMMAND.COM was found in the top directory of the <srcdir>. |
| Resolution | Delete COMMAND.COM from the <srcdir> directory and re-invoke XFORMAT. |
|  | Table 2. Troubleshooting Error Messages. |
| Error Message | INSUFFICIENT SPACE ON TARGET DEVICE |
| Explanation | The source directory specified is too large to fit onto the target. |
| Resolution | Remove some of the files from the source directory. |
| Error Message | INVALID SLOT NUMBER SPECIFIED |
| Explanation | The slot number is not a decimal number. |
| Resolution | Ensure that the slot number does not contain any alpha or special characters. |
| Error Message | INVALID SOURCE PATHNAME |
| Explanation | XFORMAT could not find the directory specified as the source pathname. |
| Resolution | Ensure that the source pathname exists. |
| Error Message | INVALID TARGET SIZE SPECIFIED |
| Explanation | An illegal character was detected in the file size portion of the /F parameter. |
| Resolution | Specify the file size using only hex characters. |
| Error Message | MORE THAN ONE SOURCE PATH WAS SPECIFIED |
| Explanation | Multiple source paths were detected on the command line. |
| Resolution | Remove one of the source paths from XFORMAT's invocation line. |
| Error Message | NO ARGUMENTS SPECIFIED |
| Explanation | The command line does not specify any operations. |
| Resolution | Execute XFORMAT with /H to display flags and options. |
| Error Message | NO SLOT SELECTED FOR THE EXM-2A |
| Explanation | XFORMAT requires specification of a slot when formatting SRAM and EXM flash memory. |
| Resolution | Rerun XFORMAT using the /P(osition) flag to indicate which EXM slot to format. |
| Error Message | NO SRAM MEMORY IN SPECIFIED SLOT |
| Explanation | The /T=S option was selected, however, no SRAM memory was detected on the card in the specified slot. |
| Resolution | Ensure the /P(osition) flag is correct or that the /T(ype) flag is specified correctly. |

Table 2. Troubleshooting Error Messages, ctd.

| Error Message | SOURCE PATHNAME TOO LONG |  |  |
| --- | --- | --- | --- |
| Explanation | Insufficient storage for the pathname string. |  |  |
| Resolution | Copy the files to a directory that is closer to the root. |  |  |
| Error Message | SLOT <n> DOES NOT CONTAIN AN ENABLED EXM-2A |  |  |
| Explanation | The slot number specified in the SRAMDISK.SYS configuration line in CONFIG.SYS does not contain an enabled EXM-2A. |  |  |
| Resolution | Enable the EXM-2A in the EXM Setup Screen, or verify that the EXM-2A is in the correct slot. |  |  |
| Error Message | SLOT NUMBER OUT OF RANGE |  |  |
| Explanation | The EXM slot number listed does not contain an EXM-2A. |  |  |
| Resolution | Ensure that the EXM slot specified contains an EXM-2A. |  |  |
| Error Message | SRAM FILE SYSTEMS ARE NOT BOOTABLE. DO NOT SPECIFY THE /S, /N OR /B PARAMETERS |  |  |
| Explanation | XFORMAT does not create bootable SRAM disks. |  |  |
| Resolution | Remove the /S, /N or /B flag. |  |  |
| Error Message | SYSTEM DOES NOT CONTAIN ONBOARD FLASH |  |  |
| Explanation | /T=O was specified for a system that does not contain onboard flash (flash on the CPU card). |  |  |
| Resolution | Repeat the command without the /T=O option. |  |  |
| Error Message | TARGET SIZE NOT SPECIFIED |  |  |
| Explanation | The /F option is ill-formed. |  |  |
| Resolution | The syntax for the /F flag is /F<filename> <size>. The filename is specified and is followed by the file size in kilobytes. |  |  |
| Error Message | UNABLE TO INITIALIZE FLASH DEVICE |  |  |
| Explanation | XFORMAT cannot write to flash memory when the write disable jumper is installed. Or, no flash memory exists. |  |  |
| Resolution | Put the JP1 jumper in the write-enable position prior to executing XFORMAT. Check that the system is properly connected to a +12V power supply. |  |  |

Table 2. Troubleshooting Error Messages, ctd.

| Error Message | UNABLE TO READ BOOT RECORD FILE |  |  |
| --- | --- | --- | --- |
| Explanation | The boot record files are not in the <srcdir> path. |  |  |
| Resolution | Copy the boot record files to the proper directory |  |  |
| Error Message | UNABLE TO READ SRAM DRIVE |  |  |
| Explanation | The SRAMDISK.SYS driver cannot read SRAM memory. |  |  |
| Resolution | Ensure the SRAM disk is formatted. The OB1 option byte must be enabled via the EXM Setup Screen. |  |  |
| Error Message | UNRECOGNIZED FLAG |  |  |
| Explanation | A mis-spelled or illegal flag was used. |  |  |
| Resolution | Check the command line to ensure only legal flags are used. |  |  |

Table 2. Troubleshooting Error Messages, ctd.

Theory of Operation

Any function or application other than XFORMAT that attempts to write to the flash disk will cause the system to lock up. Typical applications that write to disk might be Norton Utilities or PC Tools; typical DOS functions would be COPY, XCOPY, FORMAT, and FDISK.  COPY and XCOPY operate correctly from the SRAM disk.

The SRAM and flash portions of the EXM-2A are visible to the operating system as separate logical drives.  Applications that read the flash memory operate identically as if they were reading any "read only" media, such as CD-ROM disks.

DOS functions such as COPY and XCOPY will operate correctly when invoked from the flash disk.

NOTES