Appendix A:  
Replacing the Battery

The batteryxe "battery"§ powers the SRAMxe "TOD clock"§ when system power is not present.  At 60°C, the battery should have a shelf life of over four years.  In a system that is powered on much of the time and where the ambient power-off temperature is less than 60°C, the battery is estimated to have a life of 3 years.

The EXM-2A contains a "supercap" to allow the user to replace a mostly drained battery without losing SRAM contents. When replacing a battery that is not fully drained, (i.e., >2.0V) the user has 1 minute to remove and replace the battery without losing the SRAM contents. It is a good idea to make sure you have your replacement battery ready for installation before you remove the old battery.

The battery supplied with the EXM-2A is a 23mm 3V Lithium "coin" battery or equivalent (e.g. Panasonic BR2330 or Rayovac BR2335).  It is mounted near the front center of the printed circuit board, close to the front panel.  Should the batteryxe "battery"§ fail, you may obtain and install a replacement.  Figure 8 below illustrates how to change the battery.

µ §

Figure 8. Battery Replacement.

When installing the coin cell battery, be careful not to short the battery socket terminals together with the case of the battery. Doing so may cause loss of the SRAM contents.

µ §

The EXM-2A can sustain a 5 second short of the battery socket terminals without data loss if the battery being replaced is 2V or greater.