Appendix B:  
Creating Bootable Disks

From Non-Bootable Drives

In order to create a bootable disk image from a non-bootable drive, use the XFORMAT function with the /N flag.  Refer to the example below:

XFORMAT /N=6 /B=D: C:\FLASH

This example assumes that the D: drive is a non-bootable device, such as a network drive or a RAM disk, and contains the necessary system files; that DOS 6 is the operating system; and that files from the C:\FLASH directory will be copied to the system disk.

The drive specified by the /B= option must contain the system files COMMAND.COM, IO.SYS and MSDOS.SYS in addition to the BB6.00 boot block file. These files are automatically copied to the flash disk root directory when the disk is made bootable. If these system files are not present, an error message displays and XFORMAT fails.

Since the IO.SYS and MSDOS.SYS files are hidden (i.e., do not display when using a DIR command), it is necessary to unhide them so that the DOS COPY command can transfer the files. For example, to reveal the attributes set for the MSDOS.SYS file, type the following:

ATTRIB MSDOS.SYS

Letters that display include S (system file), H (hidden file) and R (read-only). To prepare the hidden file MSDOS.SYS for copying, type either of the following:

ATTRIB -s -h MSDOS.SYS

or 	ATTRIB MSDOS.SYS -s -h

Refer to the Microsoft MS-DOS User Guide and Reference or use the online help by typing ATTRIB /? at the command line for more information about the ATTRIB command and various file attributes.

It is a requirement for the IO.SYS and MSDOS.SYS files to be hidden, system, and read-only files in order for them to be bootable and/or safe from accidental damage.  Be sure to re-run ATTRIB after copying the files to the source disk to reset the system files.