# 2.  Installation

Before installing the EXM-3, unpack it and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-3, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-3 into an EXM carrier, such as the EXP-MC, is straightforward.  In the case of the EXP-MC, first ensure that the subplane has been inserted into the backplane and that the EXP-MC has been inserted into the subplane and attached to the chassis. Then remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-3 into the card guides.  Firmly press the EXM-3 front panel to ensure that the module is properly seated in the backplane and secure it with the thumbscrews.

*	make sure power to your system is off. the exm is not designed to be inserted or removed from a live system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Running the EXM Configuration Program

For the EXM-3, the configuration program simply specifies in which slot the EXM-3 resides, and whether the EXM-3, floppy controller, and SCSI controller are enabled.  This information is recorded in the EPC's nonvolatile memory and is used by the EPC's BIOS at initialization time.

To use the configuration program, you can execute it from the floppy diskette or first copy it from the floppy diskette to a hard disk.  For instance, to execute it from the floppy diskette, put the diskette in your EPC and type

a:exmcfg

The configuration program must be executed on the EPC in which you've installed the EXM-3.  If you cannot execute the program (e.g., the EXM-3 is controlling your only storage media and you are using it for the first time), there is a lower-level way to enter your configuration information via the EPC's BIOS setup screen.  In this case, skip the remainder of this section.

The configuration program first displays a screen showing what devices appear to be installed in each EXMbus slot, and what devices are described as being installed in each slot.  Follow the instructions and select the slot in which your EXM-3 is installed.

The next screen, specific to the EXM-3, asks you whether you want the EXM-3 controller enabled or not, and whether you want the floppy disk and SCSI interfaces enabled.  In almost all situations you will want to enable all three.  If you have multiple EXM-3 modules in the same system, you must not enable more than one floppy disk interface and one SCSI interface at any point in time (enabling and disabling can be done dynamically by software).

Follow the remainder of the instructions on the screen of the configuration program to save the specified configuration in the EPC's nonvolatile RAM.  After this, every time you boot or reset your EPC, the BIOS will initialize the EXM-3 in the specified slot to the specified configuration.

## Connectors

The EXM-3 has two connectors on its front panel: a standard SCSI nonshielded 50-pin connector and a 34-pin floppy disk drive connector.

The 50-pin SCSI connector on the front panel of the EXM-3 matches the definition of a single-ended nonshielded SCSI device connector in the ANSI SCSI standard.  All odd pins except pin 25 are Ground.  Pin 25 is not used and unconnected.  The even-numbered pins are defined below.

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 2 | -DB(0) | 28 | Ground |
| 4 | -DB(1) | 30 | Ground |
| 6 | -DB(2) | 32 | -ATN |
| 8 | -DB(3) | 34 | Ground |
| 10 | -DB(4) | 36 | -BSY |
| 12 | -DB(5) | 38 | -ACK |
| 14 | -DB(6) | 40 | -RST |
| 16 | -DB(7) | 42 | -MSG |
| 18 | -DB(P) | 44 | -SEL |
| 20 | Ground | 46 | -C/D |
| 22 | Ground | 48 | -REQ |
| 24 | Ground | 50 | -I/O |
| 26 | TERMPWR (see next |  |  |
|  | page) |  |  |

The 34-pin header is for connection of a floppy diskette drive.  All odd-numbered pins are Ground.  The even-numbered pins are defined below:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 2 | -DENS | 18 | -DIRC |
| 4 | not used | 20 | -STEP |
| 6 | not used | 22 | -WD |
| 8 | -IDX | 24 | -WE |
| 10 | -MO2 | 26 | -TRQ0 |
| 12 | -DS1 | 28 | -WP |
| 14 | -DS2 | 30 | -RDD |
| 16 | -MO1 | 32 | -HS |
|  |  | 34 | -DCH |

Most standard 5.25" and 3.5" drives can be used.  Assuming the drive is to be configured as the "A" drive on a straight cable,

•	If the drive has a ds (drive-select) jumper or switch, set it to 2.

•	If the drive has a mm/ms (motor-control) jumper or switch, set it to ms (motor control derived from drive-select signal).

•	If the drive has a sr/dc (disk change) jumper or switch, set it to dc.

The signals on pins 10-16 connect the drive through a straight ribbon cable as the "A" drive.  To connect a drive as the "B" drive, twist signals 10-16 in the cable (i.e., connect drive pin 10 to EXM-3 pin 16, and drive pin 12 to EXM-3 pin 14).

The EXM-3 contains one jumper position for supplying fused +5V to SCSI pin 26 (TERMPWR).  The EXM-3 is shipped without the jumper (pin 26 is unconnected).  To supply TERMPWR, install a jumper connecting the two pins shown below.

This product is designed to provide connectivity for external floppy drives and/or hard disks via ribbon cables from the front panel connectors. You should be aware that, as with all electrical cables, these ribbon cables can cause severe system problems if they short to the chassis. This condition can occur if the ribbon cable extends beyond the edge of the connector, as shown below:

µ §

To help alleviate problems, the following cables are recommended:

| Manufacturer | Floppy Connector (34 pin) | SCSI Connector |
| --- | --- | --- |
| 3M | 3414-7000 | 3425-7000 |
| Ansley | 609-3430CE | 609-5030CE |

NOTES