# 3. Operation

This chapter discusses the use of the EXM-3.

## Configuration using BIOS Setup Screen

There are some situations in which you will be unable to run the EXM configuration program (e.g., the program is on a drive plugged into the EXM-3 and you are installing the EXM-3 for the first time).  To do this, invoke the setup program by pressing the CTRL-ALT-ENTER keys at the operating-system prompt or by rebooting your EPC, waiting for the selftests to complete, and then pressing CTRL-ALT-ESC.  On the screen you should see something like

ID OB1 OB2       
 Slot 0    FC  07  00
      1    FF  00  00
      2    FF  00  00
      3    FF  00  00
      4    FF  00  00
      5    FF  00  00

This displays the data (in hexadecimal) stored in nonvolatile memory describing the EXM configuration.  ID is the EXM's ID (the ID for EXM-3 is FC, the ID for "no device expected" is FF).  OB1 and OB2 are two bytes of option information.  EXM-3 uses only the three low-order bits of OB1.

To add or change an EXM-3 configuration, the ID byte should be set to FC for the slot in which the EXM-3 resides, and OB1 should be set to one of the following:

00	EXM-3 disabled

03	SCSI enabled, floppy controller disabled

05	Floppy enabled, SCSI controller disabled

07	SCSI and floppy interfaces enabled

## Troubleshooting

Mating a disk controller to a new set of cables and the wide variety of drives available can be an exasperating experience.  If you have trouble, try the following before calling for assistance.

If you cannot boot from a floppy disk, the problem could be (1) the software configuration, (2) the drive configuration, (3) power to the drive, (4) the cable, (5) an improperly formatted diskette, or (6) the EXM-3.  Invoke the BIOS setup program and ensure that the floppy drive specification matches that of your drive, that the EXM-3 is in the proper slot, and that the EXM-3 configuration data specifies that both the EXM-3 and floppy controller are enabled.  Review the bullets at the end of the previous chapter to ensure that the drive is configured correctly.  Ensure that you are supplying power to the drive.  Check the cable connections and the cable itself.  Ensure that you have a bootable diskette (e.g., try booting from it in another PC).

If you cannot boot from or access a SCSI device, the problems listed above apply.  Invoke the BIOS setup program and ensure that you have the SCSI driver enabled.  Ensure that the EXM-3 is in the proper slot (consistent with the EXM configuration information) and that the configuration data specifies that both the EXM-3 and SCSI controller are enabled.  Ensure the SCSI boot device is configured as device 0 or 1.  Check power to the device (typically both +5V and +12V).  Check the cable connections and the cable itself.  If the trouble is booting, ensure that the device has been formatted as a system disk (which will require connecting and booting from a floppy to do so).

Some error messages that might appear and their interpretations are

Disk boot failure.  No boot disk could be found.

Diskette drives or types mismatch error.  The BIOS setup information is inconsistent with the drives detected (or absent).

Floppy disk cntrlr error.  The BIOS could not find the EXM-3 (e.g., missing or disabled).

General failure...  Usually indicates an unformatted disk or diskette.

Non-system disk.  Trying to boot from a disk or diskette that is not formatted as a bootable system disk.

SCSI controller failed.  The SCSI driver is enabled but the BIOS could not find the EXM-3 (e.g., missing or disabled).

SCSI disk failed.  The SCSI device or cable is not operational.