# 4.  Programming Interface

This chapter contains information needed to write custom software drivers for the EXM-3.  Anyone just using the EXM-3 as a BIOS-controlled interface to external disks can skip reading this chapter.

The EXM-3 defines the following registers in the I/O space.

| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O Port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 100 |
| Device ID Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  | FLEN | SCEN | CDEN | 102 |
| Configuration Option Byte 1 Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 2B0 |
| SCSI Data Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 2B1 |
| SCSI Initiator Command Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 2B2 |
| SCSI Mode Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 2B3 |
| SCSI Target Command Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 2B4 |
| SCSI Bus Status Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 2B5 |
| SCSI DMA/Interrupt Register |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2B7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Floppy Operations Register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3F2 |  |
| Floppy Command Register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3F4 |  |
| Floppy Data Register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3F5 |  |  |
| Floppy Control Register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3F7 |  |  |

The first two are standard EXM registers for device identification and configuration.  The EXM-3  responds to accesses to these ports only if EXMbus line -EXMID is asserted.  An 8-bit read from I/O address 100h returns the value FC, the device ID of the EXM-3.  A read/write configuration register appears at I/O address 102h.  The defined bits are

FLEN specifies whether the floppy diskette controller is enabled (1) or disabled (0).  If FLEN and CDEN are set, the I/O addresses 3F0-3F7 and the floppy diskette controller are active.

SCEN specifies whether the SCSI controller is enabled (1) or disabled (0).  If SCEN and CDEN are set, the I/O addresses 2B0-2B7 and the SCSI controller are active.

CDEN specifies whether the EXM-3 is enabled (1) or disabled (0).  If disabled, the EXM-3 will not respond to the 3F0-3F7 and 2B0-2B7 I/O addresses; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h, and then only if EXMbus line -EXMID is asserted.

For an explanation of the SCSI controller registers, refer to documentation on a 53C80-type SCSI controller.  For an explanation of the floppy diskette controller registers, refer to documentation on the WD37C65 controller chip.