# 1.  Getting Started

This manual contains the information you need to install and use the EXM-4 IEEE-488 interface.

The EXM-4 is fully software-compatible with the popular National Instruments GPIB-PCII IEEE-488 interface; a wide range of software packages for the GPIB-PCII can be used with the EXM-4.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-4.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 85° C |
| Humidity | operating | 5% - 95% (non-condensing) |
|  | storage | 5% - 95% (non-condensing) |
| Vibration | operating | .015"displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | .030" displacement with 5 g peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30g 11 msec duration, half-sine shock pulse |
|  | storage | 50g 11 msec duration, half-sine shock pulse |
| Electrical |  |  |
| Voltage | +5V Supply | +5V ± 5% |
| Current | max | 5V @ 1.2A |
| Miscellaneous |  |  |
| Weight |  | 3.0 ounces |

Table 1. EXM-4 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.