# 2.  Installation

Before installing your EXM-4, you should unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-4, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-4 into an EXM carrier is straightforward.  Remove any blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-4 into the card guides.  Firmly press the EXM-4 front panel to ensure that it has mated with the rear connector, and secure it with the thumbscrews. DO NOT OVERTIGHTEN.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

## Configuration Using the BIOS Setup Screen

The EXM configuration data in the EPC to which the EXM-4 is connected needs to be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles Figure 1 below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    F5  05  00
      2    FB  79  00
      3    DE  01  00
      4    ED  01  00
      5    7D  05  91

Figure 1. EXM Setup Screen.

The setup screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2 for each installed EXM.

All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present. A typical value for the EXM-4 is shown in bold letters in slot 2.

Slot, OB1 and OB2 are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-4 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 2. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-4 is FBh.

OB1 and OB2 are two bytes of option information.  OB1 is a hexadecimal value derived by combining the following:

| DMA Channel Bits 7-6 |  | Interrupt Bits 5-3 |  | I/O Base Bits 2-1 |  | EXM Enable Bit 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00 | none | 000 | none | 00 | 2B8 | 0 | disabled |
| 01 | 1 | 001 | none | 01 | 2C0 | 1 | enabled |
| 10 | 2 | 010 | IRQ9 | 10 | 2C8 |  |  |
| 11 | 3 | 011 | IRQ3 | 11 | 2D0 |  |  |
|  |  | 100 | IRQ4 |  |  |  |  |
|  |  | 101 | IRQ5 |  |  |  |  |
|  |  | 110 | IRQ6 |  |  |  |  |
|  |  | 111 | IRQ7 |  |  |  |  |

When making these selections, keep the following in mind:

·	Choose values that don't create conflicts with other EXMs or PC add-in cards.

·	Do not choose IRQ3 if you will also use the EPC's COM2 serial port.

·	Do not choose IRQ4 if you will also use the EPC's COM1 serial port.

·	Do not choose IRQ6 if you will also use a floppy disk drive.

·	Do not choose IRQ7 if you will also use the EPC's LPT1 parallel port.

·	Do not choose IRQ9 if you have another EXM or PC add-in card using IRQ2.  In the IBM PC architecture, IRQ2 and IRQ9 mean the same thing.

·	Do not choose DMA channels 2 and 3 if your system has a floppy disk drive and hard disk drive installed (e.g., an EXP-MS).

For instance, a typical OB1 value is 79h (01111001), representing DMA channel 1, IRQ7, an I/O base of 2B8, and EXM enabled.

OB2 is not used and should be set to 00h.