# 4.  Programming Interface

This chapter contains information needed to write custom software drivers for the EXM-4.  Anyone using the EXM-4 with software compatible with the National Instruments GPIB-PCII can skip reading this chapter.

The EXM-4 defines the following registers in the I/O space.

| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O Port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 100 |
| Device ID Register |  |  |  |  |  |  |  |  |
| DMA Channel |  | Interrupt |  |  | I/O | BASE | CDEN | 102 |
| Configuration Option Byte 1 Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | xx0 |
| Data-in / Byte-out Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | xx1 |
| Interrupt Status / Mask 1 Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | xx2 |
| Interrupt Status / Mask 2 Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | xx3 |
| Serial Poll Status / Mode Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | xx4 |
| Address Status / Mode Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | xx5 |

Command Pass-Through / Auxiliary Mode Register

|  |  |  |  |  |  |  |  | xx6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address 0 Register |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | xx7 |

Address 1 / End-of-String Register

The I/O port denoted XX0 represents the selected I/O base address for the EXM-4, XX1 represents this address plus one, and so on.

The first two are standard EXM registers for device identification and configuration.  The EXM-4 responds to accesses to these ports only if EXMbus line -EXMID is asserted.  An 8-bit read from I/O address 100h returns the value FB, the device ID of the EXM-4.  A read/write configuration register appears at I/O address 102h.  The bit encodings are defined in the previous chapter.

DMA channel selects the DMA channel to be used, or specifies that no DMA channel will be used.

Interrupt selects the interrupt to be generated, or specifies that no interrupt is to be generated.

I/O base  selects the place in the I/O space in which the 8-byte I/O port area of the EXM-4 appears.

CDEN specifies whether the EXM is disabled or enabled.  If disabled, the EXM will not respond to any I/O or memory addresses and will not assert an interrupt output; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h, and then only if EXMbus line -EXMID is asserted.

For details on the remaining registers, refer to documentation on the NEC 7210 GPIB controller chip.