# 2.  Installation

Before installing the EXM-5, unpack and inspect it for shipping damage.

	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-5, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

	ensure that the installation process as described herein is also performed in a static-free environment.

## Safety

Please take a moment to review these safety precautions.  They are provided for your protection and to prevent damage to the module.  This safety information applies to all operator and service personnel.

The exclamation point within an equilateral triangle is intended to alert the user to the presence of important operating and maintenance instructions in the literature accompanying the product. When you see this symbol on the EXM-5, be sure to refer to this manual.

Always disconnect all telephone lines from the wall outlets before 
	servicing or disassembling the equipment, or replacing batteries.

Never install telephone wiring during a lightning storm.

Never install telephone jacks in wet locations unless the jack is 	specifically designed for wet locations.

Never touch uninsulated telephone wires or terminals 		unless the telephone line has been disconnected at the 		network interface.

## Insertion in an EXM Carrier

Once the header pins are set, insertion of the EXM-5 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-5 into the card guides.  Firmly press the 
EXM-5 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews. Tighten with a screwdriver to lessen shock hazard.

	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

Once the EXM-5 is inserted into the carrier, turn the system on and boot it up. The EXM configuration data in the EPC to which the EXM-5 is connected now must be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function from the command line by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    DB  C1  00
      2    FA  25  00
      3    DE  00  39
      4    F5  05  00
      5    DC  F5  00

Figure 1. EXM Setup Screen.

The EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The configuration data for the EXM-5 is shown in bold in slot 2. The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-5 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

Figure 2. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

Note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module show an ID of FF and OB1/OB2 of   00 00  indicating no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-5 should be set to FAh.

OB1 should be set according to the following bit definitions:

Bits 7-6		Bits 5-3		Bits 2-1		Bit 0

unused		Interrupt		I/O base		EXM enable

000	not used	00	3F8	(COM1)	0	disabled
		001	not used	01	2F8	(COM2)	1	enabled
		010	IRQ9 (2)	10	3E8	(COM3)
		011	IRQ3	11	2E8	(COM4)
		100	IRQ4
		101	IRQ5
		110	IRQ6
		111	IRQ7

Table 2. OB1 Configuration.

For instance, a typical OB1 value is 25h (00100101), representing IRQ4, 3E8 (COM3), EXM enabled.

Note that you should use an interrupt that is not already in use by another device.

OB2 is not used and should be set to 00.

NOTES