# 1.  Getting Started

This manual contains the information you will need to install and use the EXM-6 and EXM-13 VGA video controllers.  When discussing information common to both, the term "EXM-6/13" will be used.

EXM-6 implements the full VGA video standard, as well as an extended 800x600 16-color mode.  It has a standard DB-15 VGA connector for attachment to an analog VGA or multiscan monitor.  It also has an auxiliary video connector similar to the auxiliary video extension in the IBM PS/2 and the feature connector on many VGA cards; this contains auxiliary video signals that can be used for connection to other types of displays, such as certain flat-panel displays.  EXM-6 contains 256 KB of video memory.

EXM-13 is a superset of the EXM-6.  It supports the additional "super VGA" resolutions of 640´480 256 colors, 800´600 256 colors, and 1024´768 16 colors.  It also contains an auxliliary connector, but with more signals available than in the EXM-6.  EXM-13 contains 512 KB of video memory and provides faster video performance than EXM-6.

EXM-6/13 uses certain standard memory and I/O addresses and thus cannot be used in systems with other video controllers also using these addresses.  Therefore the EXM-6/13 cannot be used in an EPC-2 or EPC-3 system, since these have integrated VGA controllers.  Also, more than one EXM-6/13 cannot be used in a single system unless your software ensures that no more than one EXM-6/13 is enabled at any point in time.

## Specifications

The following table defines the power and environmental specifications of the EXM-6 and EXM-13.

| Characteristic |  | Value |
| --- | --- | --- |
| Temperature | operating | 0 to 60°C ambient |
|  | storage | -40 to 125°C |
| Humidity | operating | 0 - 90% noncondensing |
|  | storage | 0 - 95% noncondensing |
| Altitude | operating | 10,000 ft (3000 m) |
|  | storage | 50,000 ft (15,000 m) |
| Vibration | operating | 0.015 inch (0.38 mm) P-P displacement with 2.5 g peak (max) acceleration over 5-2000 Hz |
|  | storage | 0.030 inch (0.76 mm) P-P displacement with 5.0 g peak (max) acceleration over 5-2000 Hz |
| Shock | operating | 30 g, 11 ms duration, half-sine shock pulse |
|  | storage | 50 g, 11 ms duration, half-sine shock pulse |
| Power | maximum | 10 W |
| (EXM-6) | typical | 4.5 W |
| Power | maximum | 8 W |
| (EXM-13) | typical | 3.5 W |
| Current | maximum | 5V @ 2A |
| (EXM-6) | typical | 5V @ 0.9A |
| Current | maximum | 5V @ 1.6A |
| (EXM-13) | typical | 5V @ 0.7A |
| Weight (EXM-6) |  | 4 oz (120 g) |
| Weight |  | 3.5 oz (105 g) |
| (EXM-13) |  |  |

XE "Power"§XE "Current"§XE "VMEbus:specifications"§XE "VMEbus:addressing"§XE "VMEbus:master data transfer"§XE "VMEbus:slave data transfer"§XE "VMEbus:interrupter"§XE "VMEbus:interrupt handler"§XE "VMEbus:requester"§XE "VMEbus:arbiter"§XE "VMEbus:system controller"§XE "Shock"§XE "Vibration"§XE "Altitude"§XE "Humidity"§XE "Temperature"§