# 2.  Installation

Before installing your EXM-6/13, you should unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-6/13, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-6/13 into an EXM carrier, such as the EXP-MC or the integrated EXM carriers in the EPC-4, 5, ... is straightforward.  First ensure that the subplane has been inserted into the backplane.  If an EXP-MC is being used, insert it in the desired VMEbus slot and attach it to the chassis.  Then remove any blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-6/13 into the cardguides.  Firmly press the EXM-6/13 front panel to ensure that it has mated with the rear connector, and secure it with the thumbscrews.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

## Running the Configuration Program

For the EXM-6/13, the configuration program allows you tro specify in which slot the EXM resides and whether the EXM is enabled.  In almost every situation you will find it easier to use the BIOS setup program discussed in the next chapter rather than the configuration program.

For the EXM-6 and EXM-13, the configuration program simply specifies in which slot the EXM-resides, and whether the EXM is to be enabled.  This information is recorded in the EPC's nonvolatile memory and is used by the EPC's BIOS at initialization time.

Depending on what types of system configurations you purchased, the exmcfg configuration program (an MS-DOS program) can be found on your hard disk or floppy diskettes or both.  For instance, to execute it from a floppy diskette, put the diskette in your EPC and type

a:exmcfg

The configuration program must be executed on the EPC in which you've installed the EXM-6/13.  An alternative lower-level way to enter your configuration information via the EPC's BIOS setup screen is described in the next chapter.

The configuration program first displays a screen showing what devices appear to be installed in each EXMbus slot, and what devices it expects to find in each slot.  Follow the instructions and select the slot in which your EXM-6/13 is installed.

The next screen, specific to the EXM-6/13, asks you about the only configuration option available - whether you want the EXM enabled or not.  If you have only one EXM-6/13 in the system, you should answer yes (enabled).  If you have multiple EXM-6/13 modules in the same system, you must not enable more than one of them (enabling and disabling can be done dynamically by software).

Follow the remainder of the instructions on the screen of the configuration program to save the specified configuration in the EPC's nonvolatile RAM.  After this, every time you boot or reset your EPC, the BIOS will initialize the EXM-6/13 in the specified slot to the specified configuration.

## Connecting a Monitor

A monitorxe "monitor"§ is connected via the 15-pin D connector on the EXM-6/13 front panel.  The EXM-6/13, being a VGA graphics controller, produces analog color signals, meaning that old-style EGAxe "EGA"§, CGAxe "CGA"§, and monochrome monitors cannot be used.  VGA monitors may be color or gray scalexe "gray-scale"§; the EXM-6/13 automatically senses the difference between the two.  If using a multiscan monitorxe "multiscan monitor"§, make sure to set the monitor's switch to analog (not TTL).

The monitor type cannot be detected correctly if the monitor is disconnected or powered off when the EPC is powered on or reset.  Typical symptoms are lack of color on a color monitor and an error message about lack of a suitable video adapter when invoking Windows.  If a color monitor is not detected correctly under MS-DOS, the command

mode co80

can be used to switch the controller into color mode.

Monitors that can be used are VGA-compatible monitors (i.e., those compatible with the IBM PS/2 and with PC VGA add-in cards) and multiscan (multifrequency or "multisync") monitors.  If you cannot mate your monitor to the EXM-6/13 because you have a cable with a 9-pin connector, either (1) you have a TTL monitor that is not compatible with VGA or (2) you have a multisync monitor (which are usually shipped with 9- and 15-pin cables or adapters) and are using the wrong cable.

To ensure a reliable connection, the monitor's cable should be screwed into the EXM-6/13's connector.

Connecting a monitor is not absolutely necessary; its presence or absence does not affect the operation of the system.  Also, the monitor can usually be safely connected and/or disconnected during operation.

The EXM-6/13 supports some video modes beyond the IBM-standard VGA modesxe "VGA modes"§, most importantly the 800 ´ 600 modes.  Use of these modes requires a super-VGA or multiscan monitorxe "multiscan monitor"§, as well as special software drivers.  Drivers for Microsoft Windows for 800´600´16 (EXM-6/13) and 1024´768´16 (EXM-13) are provided on a diskette.  This diskette is normally distributed with other RadiSys software; if you don't have it, call RadiSys customer support for a copy.

To install the driver, you need to run the Windows setup program, but you need to run it under DOS, not under Windows.  Exit from Windows and refer to a section in your Windows manual entitled something like "Running Setup from MS-DOS" or "Installing a Device Driver Not Supplied with Windows."  The setup program will ask for your driver diskette.

## Connectors

The DB-15 monitor connector is defined as

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Red | 9 | (key) |
| 2 | Green | 10 | Ground |
| 3 | Blue | 11 | (not used) |
| 4 | (not used) | 12 | (not used) |
| 5 | Ground | 13 | Horizontal sync |
| 6 | Ground | 14 | Vertical sync |
| 7 | Ground | 15 | (not used) |
| 8 | Ground |  |  |

The EXM-6 contains a 20-pin auxiliary connector.  Usually this connector is not used.  It provides auxiliary video signals for enhanced video control and special types of displays (e.g., flat panels).  The connector is a 3M series 3592 (or equivalent) header with 0.100" ´ 0.100" pin spacing and mates with a 3M series 3421 or equivalent connector.

The pins on the auxiliary video connector on the EXM-6 are defined as

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | EVIDEO | 11 | P6 |
| 2 | F1 | 12 | P3 |
| 3 | P7 | 13 | Ground |
| 4 | P0 | 14 | P4 |
| 5 | Ground | 15 | -BLANK |
| 6 | P1 | 16 | P5 |
| 7 | DCLK | 17 | HSYNC |
| 8 | P2 | 18 | F2 |
| 9 | EDCLK | 19 | VSYNC |
| 10 | Ground | 20 | ESYNC |

The EXM-13 contains a similar auxiliary connector.  Its 36-pin connector is a shielded female AMP 174726-3.  It mates with an AMP 174340-3 male connector with 174747 shield case kit, or equivalent.

The pins on the auxiliary video connector on the EXM-13 as well as the wire numbers on a typical flat cable, are defined below.

| Pin | Cable wire | Signal | Pin | Cable wire | Signal |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | EVIDEO | 19 | 2 | Ground |
| 2 | 3 | P7 | 20 | 4 | P0 |
| 3 | 5 | Ground | 21 | 6 | P1 |
| 4 | 7 | DCLK | 22 | 8 | P2 |
| 5 | 9 | EDCLK | 23 | 10 | Ground |
| 6 | 11 | P6 | 24 | 12 | P3 |
| 7 | 13 | Ground | 25 | 14 | P4 |
| 8 | 15 | -BLANK | 26 | 16 | P5 |
| 9 | 17 | HSYNC | 27 | 18 | Ground |
| 10 | 19 | VSYNC | 28 | 20 | ESYNC |
| 11 | 21 | Ground | 29 | 22 | Ground |
| 12 | 23 | reserved | 30 | 24 | Ground |
| 13 | 25 | reserved | 31 | 26 | Ground |
| 14 | 27 | reserved | 32 | 28 | Ground |
| 15 | 29 | Ground | 33 | 30 | Ground |
| 16 | 31 | RED | 34 | 32 | Ground |
| 17 | 33 | GREEN | 35 | 34 | Ground |
| 18 | 35 | BLUE | 36 | 36 | Ground |

The signals on the auxiliary connectors of the EXM-6 and EXM-13 are described below.

-BLANK	This input/output signal is an alternate blanking signal to the digital-to-analog converter (DAC) in the EXM-6/13.  When enabled (see ESYNC), a low input causes the DAC to drive its analog color outputs to 0V.  When disabled, this pin outputs the internal blanking signal.

BLUE	This analog output is the same as the blue output on the DB-15 connector (EXM-13 only).

DCLK	This input/output signal is the dot clock to the DAC.  When enabled (see EDCLK), it is an input whose rising edge causes the DAC to latch the digital video signals.  When disabled, it outputs the internal dot clock.

EDCLK	This input signal controls the source of the dot clock.  When high (or unconnected), the clock is internal.  When low, the DCLK input is enabled.  Also, when EDCLK is low, the internal 36 MHz oscillator is disconnected and DCLK used in its place.  While EDCLK is high or unconnected, the VGA miscellaneous output register must be configured so that it is not selected clock source 2.

EVIDEO	This input signal controls the source of the digital video information to the DAC.  When high (or unconnected), the digital signals are obtained internally.  When low, the digital signals are sourced from P0-P7 on the connector.

ESYNC	This input signal controls the source of the blanking and synchronization signals.  When high (or unconnected), they are obtained internally.  When low, the inputs -BLANK, HSYNC, and VSYNC are enabled as the source.

F1	This output signal is the logical and of DCLK and -BLANK.  It is provided for certain flat-panel displays, such as the Fujitsu FPF8060 (EXM-6 only).

F2	This output signal is not currently defined and is always high (EXM-6 only).

GREEN	This analog output is the same as the green output on the DB-15 connector (EXM-13 only).

HSYNC	This input/output signal is the horizontal sync signal on the VGA monitor connector.  When enabled (see ESYNC), it is an input and drives the horizontal sync signal on the VGA monitor connector.  When disabled, it outputs the horizontal sync signal.

P0-P7	These input/output signals are the digital video inputs to the EXM-6/13's DAC.  When enabled (see EVIDEO), they are inputs that drive the DAC.  When disabled, they output the internal digital video signals.  P0-P5 represent, respectively, blue, green, red, secondary blue, secondary green, and secondary red.

RED	This analog output is the same as the red output on the DB-15 connector (EXM-13 only).

VSYNC	This input/output signal is the vertical sync signal on the VGA monitor connector.  When enabled (see ESYNC), it is an input and drives the vertical sync signal on the VGA monitor connector.  When disabled, it outputs the vertical sync signal.

## Flat-Panel Displays

Flat-panel displays have a number of different conventions for interfacing to a VGA controller.  Most require some of the signals on the auxiliary connector.  Some connect to the analog color signals; others do not.  Some "non-color" flat panels operate best with the video controller in gray-scale mode; others have their own color-to-gray-scale conversion logic and require color mode.  The latter may require you to "trick" the EXM into thinking that an analog color monitor is connected.

The EXM-6/13 detects the type of monitor in use at the time of system reset by sensing the three analog color signals on the DB-15 VGA connector.  If it senses a 75 ohm load on all three color signals, it initializes to color mode.  All other conditions initialize the controller to gray-scale mode.

If you are using a flat-panel display that requires color mode (e.g., it does its own conversion) and either doesn't connect to the analog signals or doesn't provide the 75-ohm load on the analog signals, you may need to build a terminating connector that plugs into the DB-15 connector.  The terminating connector contains three 75-ohm resistors.  The resistors connect the red, green, and blue color signals (pins 1-3) to one of the ground pins (e.g., pin 5).