# 3.  Operation

## Configuration Using BIOS Setup Screen

As an alternative to running the EXM configuration program, you can also establish the EXM's configuration using the BIOS setup screen.  To invoke the setup program, press CTRL+ALT+ESC at the operating-system prompt, or reboot your EPC, wait for the selftests to complete, and press CTRL+ALT+ESC.  Once in the setup program, press the designated function key to invoke the EXM menu.  On the screen you should see something like

EXM Setup Screen

           ID OB1 OB2       
 Slot 0    EF  01  00
      1    FF  00  00
      2    FF  00  00
      3    FF  00  00
      4    FF  00  00
      5    FF  00  00

This displays the data (in hexadecimal) stored in nonvolatile memory describing the EXM configuration.  ID is the EXM's ID (the ID for EXM-6 is EF; the ID for the EXM-13 is ED; the ID for "no device expected" is FF).  OB1 and OB2 are two bytes of option information.  EXM-6/13 uses only one bit of OB1.

To add or change an EXM-6 configuration, the ID byte for the slot in which the EXM-6 resides should be set to EF, and OB1 should be set according to the following bit definitions:

| Bits 7-1 | Bit 0 |
| --- | --- |
| unused | EXM enable |
|  | 0 disabled |
|  | 1 enabled |

To add or change an EXM-13 configuration, the above applies except that the ID byte is ED.

## Default Initialization of Video EXMs

For a variety of reasons, the BIOS in EPCs automatically enables a video EXM if it finds that none are enabled.  The primary reason is the chicken-and-egg problem of configuring the EXM the first time; one needs to be able to see the video output of the setup program in order to configure the video EXM.

If the BIOS finds no enabled video EXM, it searches for the first video EXM by slot and enables it.  You should then use the setup program to configure the EXM properly to avoid receiving an exm configuration error message every time the system is booted.