4.  Programming Interface

This chapter contains information needed to write custom software drivers for the EXM-6/13.  Anyone using the EXM with software compatible with standard PC VGA, EGA, or CGA video adapters can skip reading this chapter.

The EXM-6 and EXM-13 are mapped into both the memory space and I/O space.  In the memory space the mapping is

| Range |  | Content |
| --- | --- | --- |
| 000A0000 | 000BFFFF | Video RAM |
| 000C0000 | 000C7FFF | Video BIOS ROM |

The EXM-6 and EXM-13 define the following registers in the I/O space.

| Port | Functional group | Usage |  |  |
| --- | --- | --- | --- | --- |
| 100 | EXM Configuration | Device ID |  |  |
| 102 |  | Option byte 1 |  |  |
| 3B4 | Video controller | CRT index register (mono) |  |  |
| 3B5 |  | CRT ctlr data reg (mono) |  |  |
|  |  | 0 | Horizontal total register |  |
|  |  | 1 | Horiz display enable end |  |
|  |  | 2 | Start horiz blanking |  |
|  |  | 3 | End horizontal blanking |  |
|  |  | 4 | Start horiz retrace pulse |  |
|  |  | 5 | End horizontal retrace |  |
|  |  | 6 | Vertical total register |  |
|  |  | 7 | Overflow |  |
|  |  | 8 | Preset row scan |  |
|  |  | 9 | Max scan line/others |  |
|  |  | A | Cursor start |  |
|  |  | B | Cursor end |  |
|  |  | C | Start address high |  |
|  |  | D | Start address low |  |
|  |  | E | Cursor location high |  |
|  |  | F | Cursor location low |  |

| Port | Functional group | Usage |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10 | Vertical retrace start |  |
|  |  | 11 | Vertical retrace end |  |
|  |  | 12 | Vert display enable end |  |
|  |  | 13 | Offset |  |
|  |  | 14 | Underline location |  |
|  |  | 15 | Start vertical blank |  |
|  |  | 16 | End vertical blank |  |
|  |  | 17 | CRTC mode control |  |
|  |  | 18 | Line compare |  |
| 3B8 |  | Mode control reg (mono) (W) |  |  |
| 3B9 |  | Preset light pen (mono) (W) |  |  |
| 3BA |  | Input status reg 0 (mono) (R) |  |  |
|  |  | Feature control (mono) (W) |  |  |
| 3BB |  | Clear light pen (mono) (W) |  |  |
| 3BF |  | Hercules register |  |  |
| 3C0 |  | Attribute index register |  |  |
|  |  | Attribute data register (W) |  |  |
| 3C1 |  | Attribute data register (R) |  |  |
|  |  | 0X | Palette registers |  |
|  |  | 10 | Attribute mode control |  |
|  |  | 11 | Overscan control |  |
|  |  | 12 | Color plane enable |  |
|  |  | 13 | Horizontal PEL panning |  |
|  |  | 14 | Color select |  |
| 3C2 |  | Misc output register (W) |  |  |
|  |  | Input status register 0 (R) |  |  |
| 3C3 |  | Video subsystem enable reg |  |  |
| 3C4 |  | Sequencer index register |  |  |
| 3C5 |  | Sequencer data register |  |  |
|  |  | 0 | Reset register |  |
|  |  | 1 | Clocking mode register |  |
|  |  | 2 | Map mask register |  |
|  |  | 3 | Character map select reg |  |
|  |  | 4 | Memory mode register |  |
| 3CA |  | Feature control register (R) |  |  |
| 3CC |  | Misc output register (R) |  |  |
| 3CE |  | Graphics controller index reg |  |  |
| 3CF |  | Graphics data register |  |  |
|  |  | 0 | Set/reset |  |
|  |  | 1 | Enable set/reset |  |
|  |  | 2 | Color compare |  |
|  |  | 3 | Data rotate |  |
|  |  | 4 | Read map select |  |
|  |  | 5 | Graphics mode |  |
|  |  | 6 | Miscellaneous |  |
|  |  | 7 | Color don't care |  |

| Port | Functional group | Usage |  |  |
| --- | --- | --- | --- | --- |
|  |  | 8 | Bit mask |  |
|  |  | 9 | Address offset A (PR0A) |  |
|  |  | A | Address offset B (PR0B) |  |
|  |  | B | Memory size (PR1) |  |
|  |  | C | Video select (PR2) |  |
|  |  | D | CRT control (PR3) |  |
|  |  | E | Video control (PR4) |  |
|  |  | F | Lock/status (PR5) |  |
| 3D4 |  | CRT index register (color) |  |  |
| 3D5 |  | CRT controller data (color) |  |  |
|  |  | 0 | Horizontal total |  |
|  |  | 1 | Horiz display enable end |  |
|  |  | 2 | Start horizontal blanking |  |
|  |  | 3 | End horizontal blanking |  |
|  |  | 4 | Start horiz retrace pulse |  |
|  |  | 5 | End horizontal retrace |  |
|  |  | 6 | Vertical total register |  |
|  |  | 7 | Overflow |  |
|  |  | 8 | Preset row scan |  |
|  |  | 9 | Max scan line/others |  |
|  |  | A | Cursor start |  |
|  |  | B | Cursor end |  |
|  |  | C | Start address high |  |
|  |  | D | Start address low |  |
|  |  | E | Cursor location high |  |
|  |  | F | Cursor location low |  |
|  |  | 10 | Vertical retrace start |  |
|  |  | 11 | Vertical retrace end |  |
|  |  | 12 | Vert display enable end |  |
|  |  | 13 | Offset |  |
|  |  | 14 | Underline location |  |
|  |  | 15 | Start vertical blank |  |
|  |  | 16 | End vertical blank |  |
|  |  | 17 | CRTC mode control |  |
|  |  | 18 | Line compare |  |
| 3D8 |  | Mode control reg (color) (W) |  |  |
| 3D9 |  | Color select register (W) |  |  |
| 3DA |  | Input status reg 0 (color) (R) |  |  |
|  |  | Feature control (color) (W) |  |  |
| 3DB |  | Clear light pen (color) (W) |  |  |
| 3DC |  | Preset light pen (color) (W) |  |  |
| 3DE |  | AT&T / M24 (W) |  |  |

All but the first two registers are standard VGA registers.  For further explanation consult the manual of the Western Digital Paradise VGA1A video controller chip (EXM-6), Western Digital WD90C11 video controller chip (EXM-13), the text EGA/VGA A Programmer's Reference Guide by B. D. Kliewer (McGraw-Hill), or the text Programmer's Guide to PC and PS/2 Video Systems by R. Wilton (Microsoft Press).  The other two registers are

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Reg (EXM-6) | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Reg (EXM-13) | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg |  |  |  | Cden | 102 |
| --- | --- | --- | --- | --- | --- |

These are standard EXM registers for device identification and configuration.  The EXM-6/13 responds to accesses to these ports only if EXMbus line -EXMID is asserted.  An 8-bit read from I/O address 100h returns the value EF or ED, the device IDs of the EXM-6 and EXM-13.  A read/write configuration register appears at I/O address 102h.

CDEN specifies whether the EXM is disabled or enabled.  If disabled, the EXM will not respond to any I/O or memory addresses and will not assert an interrupt output; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h, and then only if EXMbus line -EXMID is asserted.  (Note that certain EPCs will automatically enable the first video controller found, independent of how CDEN is set.)