# 1.  Product Description

The EXM-10A is a "universal" Ethernet controller that works in standard 8-bit and faster 16-bit mode, and emulates both the Western Digital WD8003E and WD8013 controllers and the Novell NE2000 and NE2000+ controllers.

There are no jumpers on the EXM-10A. Selection of compatibility modes and parameters such as emulation mode, IRQ, and base address is made using the Autoset setup program, included on the EXM-10A distribution diskettes.

The two distribution diskettes provided with the EXM-10A also include drivers for IPX, ODI, NDIS and FTP's packet driver interface. Operating systems supported by these drivers are DOS, Windows and OS/2. Many other operating systems, such as LynxOS and SCO UNIX, support the EXM-10A via built-in drivers.

The EXM-10A is based on National Semiconductor's AT/LANTICÔ chip, a highly integrated Ethernet controller IC. The chip contains an AT-bus interface, the equivalent of the National Semiconductor chipset (DP8390 and DP8391) and an integrated 10-base-T transceiver. A 28-pin DIP socket is located on the EXM-10A for the optional installation of a boot ROM. Standard boot ROMs are available from Novell or other network vendors.

On the front panel of the EXM-10A there are transceivers for 10-base-T (twisted pair) and 10-base-2 (thin Ethernet) and an AUI connector for external transceivers to thick Ethernet or other media. A dual LED indicates when the EXM-10A is transmitting or receiving data.

Before configuring the EXM-10A, determine if the system requires 8-bit or 16-bit data width settings; if Western Digital or NE2000 emulation is preferred; and several other setup parameters. It is a good idea to read (or at least skim) this entire manual before starting. In addition, reference the installation manuals that accompany the operating system or higher-level networking software before proceeding.

## Specifications

The following table defines the power and environmental specifications of the EXM-10A.

| Characteristic |  | Value |
| --- | --- | --- |
| Temperature | operating | 0 to 60°C ambient |
|  | storage | -40 to 85°C |
| Humidity | operating | 5% to 95% non condensing |
|  | storage | 5% to 95% non condensing |
| Altitude | operating | 10,000 ft (3000 m) |
|  | storage | 50,000 ft (15,000 m) |
| Vibration | operating | 0.015 inch (0.38 mm) Peak to Peak displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | 0.030 inch (0.76 mm) P-P displacement with 5.0 g peak (max) acceleration over 5-2000 Hz |
| Shock | operating | 30 g, 11 ms duration, half-sine shock pulse |
|  | storage | 50 g, 11 ms duration, half-sine shock pulse |
| Power | maximum | 2.5 W |
|  | typical | 1.8 W |
| Current | maximum | +5V @ 0.50A |
|  | typical | +5V @ 0.36A |
| Weight |  | 4.3 oz. |

Table 1. EXM-10A Environmental and Electrical Specifications.