# 3.  Connectors

## Connecting to the Network

If using thick Ethernet, connect the transceiver cable to the DB-15 connector on the EXM-10A front panel.

If using thin Ethernet, connect the cable to the BNC connector on the front panel. This must be done with a "T" connector.  If this EXM-10A is not the last station on the cable, the T connector should connect to two coaxial cables.  If this EXM-10A is the last station, a terminator should be connected to one side of the "T" connector.

If using twisted-pair Ethernet (10-base-T), connect the cable to the  RJ-45 connector.

Note that only one of these connections should be made to the EXM-10A at any one time.

## DB-15 Connector

A 15-pin connector is provided on the front panel for connection to an external MAU for thick Ethernet.  The connector is an AMP 745782-4 or equivalent socket and is defined in the following table:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | GND | 9 | CD- |
| 2 | CD+ | 10 | TX- |
| 3 | TX+ | 11 | GND |
| 4 | GND | 12 | RX- |
| 5 | RX+ | 13 | +12V Fused |
| 6 | GND | 14 | GND |
| 7 | No connect | 15 | No connect |
| 8 | GND |  |  |

Table 6. Thick Ethernet Connector Pin-out.

Notes:

1.	The minus sign next to a signal indicates active low.

2.	The plus sign next to a signal indicates active high.

3. 	+12V is fused at 2A.

## BNC Connector

An industry-standard BNC connector is provided for thin Ethernet connection.

Figure 3. BNC Connector.

## RJ-45 Connector

The DTE RJ-45 phone jack that supplies the 10-Base-T interface to the EXM-10A is defined in the following table:

| RJ-45 | Signal |
| --- | --- |
| 1 | No connect |
| 2 | No connect |
| 3 | Rx+ |
| 4 | No connect |
| 5 | No connect |
| 6 | Rx- |
| 7 | Tx- |
| 8 | Tx+ |

Table 7. RJ-45  Phone Jack Pin-out.