# 2.  Installation

Before installing your EXM-11, you should unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-11, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-11 into an EXM carrier, such as the EXP-MC, is straightforward.  In the case of the EXP-MC, first ensure that the subplane has been inserted into the backplane and that the EXP-MC has been inserted into the subplane and attached to the chassis.  Then remove any blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-11 into the cardguides.  Firmly press the EXM-11 front panel to ensure that it has mated with the rear connector, and secure it with the thumbscrews.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

## Connectors

The EXM-11 has two standard BNC connectors labelled gate and signal.  Both are standard TTL inputs.  When one input is captured as a result of a trigger edge in the other input, the captured input should obey setup and hold times of 50 ns relative to the trigger edge.