# 3.  Operation

## Configuration Using BIOS Setup Screen

The EXM-11 has programmed configuration options that eliminate the jumpers and DIP switches commonly found on interface cards.  The options are

·	the interrupt line to be used

·	the range of I/O addresses to be used

·	the DMA channel to be used

·	whether the EXM-11 should be enabled or disabled at EPC initialization

You can establish the EXM-11's configuration using the BIOS setup screen.  To invoke the setup program, press CTRL+ALT+ESC at the operating-system prompt, or reboot your EPC, wait for the selftests to complete, and press CTRL+ALT+ESC.  Once in the setup program, depress the designated function key to invoke the EXM menu.  On the screen you should see something like

EXM Setup Screen

           ID OB1 OB2       
 Slot 0    F4  81  00
      1    FF  00  00
      2    FF  00  00
      3    FF  00  00
      4    FF  00  00
      5    FF  00  00

This displays the data (in hexadecimal) stored in nonvolatile memory describing the EXM configuration.  ID is the EXM's ID (the ID for EXM-11 is F4, the ID for "no device expected" is FF).  OB1 and OB2 are two bytes of option information.  EXM-11 uses OB1 but not OB2.

To add or change an EXM-11 configuration, the ID byte for the slot in which the EXM-11 resides should be set to F4, and OB1 should be set according to the following bit definitions:

Bits 7-6

DMA channel

Bits 5-3

Interrupt

Bits 2-1

I/O Base

Bit 0

EXM  enable

00

DMA disabled, 8-bit FIFO transfers

000

none

00

2B8

0

disabled

01

DMA disabled, 16-bit FIFO transfers

001

reserved

01

2C0

1

enabled

10

channel 5

010

reserved

10

2C8

11

channel 6

011

IRQ3

11

2D0

100

reserved

101

IRQ5

110

IRQ11

111

IRQ15

For instance, a typical OB1 value is 81 (10000001), representing DMA channel 5, interrupts disabled, an I/O base of 2B8, EXM enabled.  You need to chose values that don't create conflicts with other EXMs or PC add-in cards (e.g., the EXM-4 has the same four choices of base addresses).  Do not choose IRQ3 if the EPC's COM2 serial port will be used; do not choose IRQ4 if the COM1 serial port will be used.