# 2.  Installation

Before installing the EXM-12, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-12, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-12 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-12 into the card guides.  Firmly press the EXM-12 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

The EXM configuration data in the EPC to which the EXM-12 is connected needs to be modified to recognize and enable the card and select from the available options.  From the command line, invoke the BIOS setup function by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles Figure 1 below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    DB  C1  00
      2    F3  05  00
      3    DE  00  39
      4    ED  01  00
      5    DC  F5  91

Figure 1. EXM Setup Screen.

The setup screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2 for each installed EXM.

Note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present. A typical value for the EXM-12 is shown in bold letters in slot 2.

Slot, OB1 and OB2 are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-12 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 2. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-12 should be set to F3h.

OB1 is a hexadecimal value derived by combining the following:

Bits 7 through 4 are unused.

DMA channel 1 is configured using bit 3.

The interrupt is configured using bits 2-1.

The card is enabled or disabled using bit 0.

OB2 is not used.

| Bit 3 DMA Channel 1 | Bits 2-1 Interrupt | Bit 0 EXM enable |
| --- | --- | --- |
| 0 DMA Disabled 1 DMA Enabled | 00 none 01 IRQ9 10 IRQ5 11 IRQ15 | 0 Disabled 1 Enabled |

Table 1. OB1 Settings.

For instance, a tyical OB1 value is 05h (00000101), representing DMA is disabled, interrupt 5 is enabled, and the EXM-12 is enabled. When selecting options, take care that there are no conflicts with other EXM modules. For instance, many of the other EXM modules have programmable interrupts and DMA channels. If more than one device is configured for a given interrupt, unstable conditions could result.