# 3. Prototyping

## Schematics

Please refer to the schematic diagram of the EXM-12 at the end of this manual. The distribution diskette contains OrCAD-compatible schematic files as well.

## PAL Equations

The EXM-12 contains three PALS, two of which are socketed DIP parts which may be replaced by the user.  The EXM-12 is shipped with two installed preprogrammed PALs containing a default configuration.  The third PAL, which is not user changeable, implements the EXM-12 configuration option byte register.

One socketed PAL, U1, is intended for I/O port address decode.  There are two I/O address ranges: 0-3FF and 8000-83FF. U1 is connected to the following signals shown in Table 2.

| Inputs | Outputs | I/Os |
| --- | --- | --- |
| SA[15,9..0] | SDEN1- | IOSPARE[2..1]- |
| AEN | SDDIR1- |  |
| IOR- | CS_10X- |  |
| IOW- | CS_102- |  |
| CDEN | IOSPARE0- |  |
| ACMID- |  |  |

Table 2. U1 Signals.

Another socketed PAL, U2, is intended for memory address decode. The memory address range is 010000 - FFFFFF in 64K byte blocks. U2 is connected to the following signals.

| Inputs | Outputs |  |  | I/Os |
| --- | --- | --- | --- | --- |
| LA[23..17],SA16 |  | MEMCS16- | MEMSPARE[4..1]- |  |
| DACK- |  | SDDIR- |  |  |
| MEMW- |  | SDEN- |  |  |
| MEMR- |  | MEMSPARE0- |  |  |
| SDDIR1 |  |  |  |  |
| SDEN1- |  |  |  |  |
| CDEN |  |  |  |  |

Table 3. U2 Signals.

Please refer to the files included on the distribution diskette for the PAL equations. For example, the file IODEC.ABL contains the following equation:

sddir1 = ior;

sden1 = (cden & !aen & (ioadr >= ^h2C0) & (ioadr <= ^h2C7) & (ior # iow))

# !aen & ior & (ioadr == ^h100) & acmid

# (!aen & (ioadr == ^h102) & acmid & (ior # iow));

cs_10X = !aen & (ior & (ioadr == ^h100) # (ioadr == ^h102)) & acmid;

iospare0 = cden & !aen & (ioadr >= ^h2C0) & (ioadr <= ^h2C7);

iospare1 = 1;

iospare2 = 1;

When modifying the PAL in locations U1 and U2, it is important to preserve certain minterms in the equations for CS_10X, SDDIR1, SDEN1, SDDIR and SDEN. These signals control logic on the EXM-12 to enable the EXM-12 ID and option byte registers. There are specific notes in the source code that describe these issues.

## Default PAL Codes

Default PAL codes do not implement the BALE signal. This signal is not necessary for I/O mapped boards, but is necessary for memory-mapped boards.

## Power Consumption

The maximum power consumption of an unpopulated EXM-12 as shipped is 5V @ 0.6A.