4. The EXM Expansion

## Interface

The EXM Expansion Interface is used to provide local expansion of embedded PCs within the context of a larger system architecture. Electrically, the EXM expansion interface is nearly identical to the 16-bit PC/AT Industry Standard Architecture bus (ISA bus). The differences include the following:

1.	RESETDRV is negative true in the EXM expansion interface and
	is positive true in the ISA bus.

2.	DMA channel 7, which consists of signals DRQ7 and DACK7, is
		not present in the EXM expansion interface.

3.	The signals SA[17...19] are not present in the EXM expansion 
		interface.

4.	The addition of a signal line (EXMID) for EXM module
		identification.

5.	The signal SBHE is negative true in both the EXM expansion
		interface and the ISA bus. Some versions of the ISA bus 
		documentation from IBM incorrectly define this signal as a 
		positive 	true signal.

The EXMID line allows the individual expansion modules to be uniquely addressed at power-up, and facilitates software configuration as an alternative to jumpers and switches. This software configuration feature is incorporated into the EXM expansion interface is such a way that unmodified PC add-in card software can be used by EXM expansion modules. The registers that use this feature are described later in this chapter.

## EXM Expansion Interface Signal Definitions and Pin-out

The EXM expansion interface signals use the same names and timing as the IBM PC/AT bus signals. The only exceptions to this are the -RESETDRV signal, which is inverted in the EXM expansion interface, and the -EXMID signal, which is unique to the EXM expansion interface.

The -EXMID signal is routed to each slot and asserted by the CPU board depending on the value of register 96. In addition, the -EXMID signal is routed to each EXM slot to access the ID register (address 100H) and the programmable option select registers (addresses 102H and 103H). The -EXMID and -RESETDRV registers are used to perform module identification and configuration under software.

Refer to the IBM Technical Reference: Personal Computer AT document number 6139362 pages 1-31 through 1-37 for the PC/AT bus signal descriptions.

Table 4 provides the pin-out for the EXM expansion connector. The A row is on the component side of the EXM module, where it is populated with PALs, gates, and capacitors. Pin A1 is located on the bottom of the card, when the card is inserted vertically into the expansion interface with the label oriented properly. The B side of the EXM-12 is populated only with smaller resistors and capacitors.

| Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | +12V | B1 | GND | A30 | -DACK5 | B30 | -IOR |
| A2 | reserved | B2 | reserved | A31 | -DACK3 | B31 | -SMEMW |
| A3 | +5V | B3 | GND | A32 | -DACK2 | B32 | -SMEMR |
| A4 | +5V | B4 | GND | A33 | reserved | B33 | GND |
| A5 | SD15 | B5 | SD14 | A34 | -DACK1 | B34 | -MEMW |
| A6 | SD13 | B6 | SD12 | A35 | -DACK0 | B35 | -MEMR |
| A7 | SD11 | B7 | SD10 | A36 | -SBHE | B36 | BALE |
| A8 | -12V | B8 | GND | A37 | +5V | B37 | CLK |
| A9 | SD9 | B9 | SD8 | A38 | AEN | B38 | GND |
| A10 | SD7 | B10 | SD6 | A39 | LA23 | B39 | LA22 |
| A11 | SD5 | B11 | SD4 | A40 | LA21 | B40 | LA20 |
| A12 | +12V | B12 | GND | A41 | LA19 | B41 | LA18 |
| A13 | SD3 | B13 | SD2 | A42 | LA17 | B42 | SA16 |
| A14 | SD1 | B14 | SD0 | A43 | TC | B43 | GND |
| A15 | IRQ15 | B15 | IRQ14 | A44 | SA15 | B44 | SA14 |
| A16 | IRQ12 | B16 | GND | A45 | SA13 | B45 | SA12 |
| A17 | IRQ11 | B17 | IRQ9 | A46 | key | B46 | key |
| A18 | IRQ7 | B18 | IRQ6 | A47 | key | B47 | key |
| A19 | IRQ5 | B19 | IRQ4 | A48 | SA11 | B48 | SA10 |
| A20 | +5V | B20 | IRQ3 | A49 | SA9 | B49 | SA8 |
| A21 | -EXMID | B21 | -RESETDRV | A50 | OSC | B50 | GND |
| A22 | -IOCHCK | B22 | GND | A51 | SA7 | B51 | SA6 |
| A23 | DRQ6 | B23 | IOCHRDY | A52 | SA5 | B52 | SA4 |
| A24 | DRQ5 | B24 | -0WS | A53 | SA3 | B53 | SA2 |
| A25 | DRQ3 | B25 | -IOCS16 | A54 | SA1 | B54 | SA0 |
| A26 | DRQ2 | B26 | -MEMCS16 | A55 | +5 | B55 | GND |
| A27 | DRQ1 | B27 | -REFRESH | A56 | +5 | B56 | GND |
| A28 | DRQ0 | B28 | GND | A57 | reserved | B57 | reserved |
| A29 | -DACK6 | B29 | -IOW | A58 | +12 | B58 | GND |

Table 4. EXM Signal Pin-out.

A subset of the EXM expansion interface, along with several other useful signals, are routed to two header locations (JP3 and JP4) just in front of the card edge connector. Refer to Figure 3 on page 13. These header locations can be used to wire the signals to other prototype logic that the user wishes to install. Users can either connect directly to the holes or add a header and wire wrap from these signals to the user logic. Table 4 defines the signal pin-out for these headers.

| JP4 |  |  |  | JP3 |  |
| --- | --- | --- | --- | --- | --- |
| Pin | Signal | Pin | Signal | Pin | Signal |
| 1 | SA1 | 29 | SA0 | 1 | SA2 |
| 2 | SA4 | 30 | SA3 | 2 | SA5 |
| 3 | SA7 | 31 | SA6 | 3 | OSC |
| 4 | SA9 | 32 | SA8 | 4 | SA10 |
| 5 | SA12 | 33 | SA11 | 5 | SA13 |
| 6 | SA15 | 34 | SA14 | 6 | TC |
| 7 | LA17 | 35 | SA16 | 7 | LA18 |
| 8 | LA20 | 36 | LA19 | 8 | LA21 |
| 9 | LA23 | 37 | LA22 | 9 | AEN |
| 10 | -SBHE | 38 | SCLK | 10 | BALE |
| 11 | -MEMR | 39 | -DACK0 | 11 | -MEMSPARE3 |
| 12 | -IOSPARE0 | 40 | -MEMW | 12 | -SMEMR |
| 13 | -SMEMW | 41 | DRQ | 13 | -DACK5 |
| 14 | -IOSPARE1 | 42 | -IOR | 14 | -IOW |
| 15 | -IOSPARE2 | 43 | DRQ0 | 15 | -REFRESH |
| 16 | MEMCS16 | 44 | USR0 | 16 | -MEMSPARE2 |
| 17 | DRQ5 | 45 | -IOCS16 | 17 | -0WS |
| 18 | IOCHRDY | 46 | USR1 | 18 | -IOCHCK |
| 19 | -RESETDRV | 47 | -EXMID | 19 | CDEN |
| 20 | IRQ4 | 48 | USR2 | 20 | -MEMSPARE0 |
| 21 | IRQ11 | 49 | INT | 21 | -MEMSPARE1 |
| 22 | -MEMSPARE4 | 50 | USR3 | 22 | -DACK |
| 23 | D0 | 51 | D1 | 23 | D3 |
| 24 | D5 | 52 | D2 | 24 | D4 |
| 25 | D6 | 53 | D7 | 25 | SD9 |
| 26 | -12V | 54 | SD8 | 26 | SD11 |
| 27 | SD13 | 55 | SD10 | 27 | SD12 |
| 28 | SD14 | 56 | SD15 | 28 | +12 |

Table 5. Prototype Interface Connector.

In the preceding table, the following signals are buffered versions of the EXM Expansion Interface signals: TC, AEN, SCLK, BALE, -SBHE, -MEMR, -MEMW, 
-SMEMR, -RESETDRV. The signals D0-D7 are buffered by a bi-directional buffer. The signals USR0-USR3, MEMSPARE0-MEMSPARE4, IOSPARE0-IOSPARE2, 
-DACK, INT, DRQ, and CDEN connect to logic or spare PAL pins on the EXM-12.

The remaining signals are directly connected to the EXM expansion interface edge connector.

Note that pin 1 on the PALs and the capacitors and on JP1, JP2, JP3 and JP4 is always denoted by a squared outline around the round hole. Similarly, pin 1 on the socketed PALs is always denoted by a squared outline. Squared holes on the capacitors are +5V, while round holes are ground.

Refer to Figure 3.

Figure 3. EXM-12 Layout.

## Using the EXM Expansion Interface

1.  For all I/O reads and writes, be sure to gate the decode with AEN deasserted. This ensures that DMA activity will not trigger an I/O access to I/O ports defined on the prototype card. Sample equations are provided in Chapter 3.

2.  When generating MEMCS16, only use addresses LA17-LA23. The timing of SA0-SA16 is too late to guarantee sufficient setup time to the EXM expansion interface controller chip on the CPU board.

3.   Take care when configuring the ports on the EXM-12 that the I/O addresses and memory addresses for the port do not conflict with addresses used by the CPU or other EXMs in the system. Please refer to the respective hardware reference manuals to determine potential conflicts. If there are conflicts, it is possible that permanent damage could occur to the components of the EPC or other EXM products. For instance, the EXM-12 may conflict with the EXM-10A if one enables the EXM-10A buffer memory in the D-page. This is because the default configuration for the EXM-12 also has the D-page enabled.

4.  Gate all functions on the prototype card with the CDEN signal. In other words, disable all I/O regions, memory regions, interrupt generation, and DMA usage if CDEN is not asserted.

5.  When decoding both I/O and memory addresses on the EXM-12, take care that the control strobes (e.g., IOR, MEMR) are gated with any data enables to the EXM expansion interface. Otherwise, it is possible that memory accesses can cause I/O decodes and vice versa, causing conflicts with other modules.

## Programming Interface

The EXM-12 defines the following registers in the I/O space.

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Register | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg | User-specified |  | Drq | Interrupt | Cden | 102 |
| --- | --- | --- | --- | --- | --- | --- |

Figure 4. EXM-12 Registers in I/O Space.

Registers 100 and 102 are standard EXM registers for device identification and configuration. The EXM-12 responds to accesses to these ports only if EXM expansion interface line -EXMID is asserted. An 8-bit read from I/O address 100h returns the value F3, the device ID of the EXM-12. A read/write configuration register appears at I/O address 102h. The bit encodings are defined in Chapter 2.

User Specified bits can be used for any purpose that the user wants. These bits are read/write bits and the outputs from the register can be used to control other logic on the prototype card. These bits are available as signals USR0-USR3 as defined in Table 5 on page 12.

Drq specifies that DMA channel 1 is disabled or enabled. To use the DMA logic, the user must attach a signal to the DRQ input.

Interrupt selects the interrupt to be generated, or specifies that no interrupt is to be generated as defined in Chapter 2. To use the interrupt logic, the user must attach the interrupt source to the INT input.

CDEN specifies whether the EXM is disabled or enabled. If disabled, the EXM will not respond to any I/O or memory addresses and will not assert an interrupt output; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h, and then only if -EXMID is asserted.

NOTES