# 1.  Product Description

The EXM-13A expansion module is a Super VGA video controller using the NCR 77C22E+ controller chip.  Software diskettes with video drivers and utilities are included for resolutions beyond the standard 640 x 480 x 16 colors; up to 1280 x 1024 x 256 colors are supported.  The EXM-13A is available in two memory sizes: 1 MByte, which supports resolutions up to 1024 x 768, and 2 MBytes, which supports resolutions up to 1280 x 1024.

The EXM-13A implements the full VGA video standard, as well as a number of "Super VGA" resolutions.  It has a standard DB-15 VGA connector for attachment to an analog VGA or multiscan monitor.  It also has an auxiliary video connector similar to the auxiliary video extension in the IBM PS/2 and the feature connector on many VGA cards; this contains auxiliary video signals that can be used for connection to other types of displays, such as certain flat-panel displays.

For current users of the EXM-13 video adapter using standard VGA mode 
(640 x 480 x 16), the EXM-13A is a "drop-in" replacement.  No additional drivers are required for this mode.  The user may notice a slight delay on power-up because the EXM-13A has both more memory to check and performs a more extensive test of that memory.

The EXM-13A uses standard video memory and I/O addresses and thus cannot be used in systems with other video controllers also using these addresses.  Therefore the EXM-13A cannot be used in an EPC-2 or EPC-3 system, since these have integrated VGA controllers.  Also, more than one video controller cannot be used in a single system unless the application software ensures that no more than one video controller is enabled at any point in time.

WARNING!

do not plug in any cable or connector into the front panel connectors while the system is powered up. in general, electronics equipment is not designed to withstand potential damage that could arise from fluctuations in power. never plug in a serial or parallel device, keyboard, transceiver, monitor or other component while the system is ON.

## Specifications

The following table defines the power and environmental specifications of the 
EXM-13A.

| Environmental |  |  |
| --- | --- | --- |
| Temperature | operating | 0 to 60°C ambient |
|  | storage | -40 to 85°C |
| Humidity | operating | 5 - 90% noncondensing |
|  | storage | 5 - 95% noncondensing |
| Altitude | operating | 10,000 ft (3000 m) |
|  | storage | 50,000 ft (15,000 m) |
| Vibration | operating | 0.015 inch (0.38 mm) P-P displacement with 2.5 g peak (max) acceleration over 5-2000 Hz |
|  | storage | 0.030 inch (0.76 mm) P-P displacement with 5.0 g peak (max) acceleration over 5-2000 Hz |
| Shock | operating | 30 g, 11 ms duration, half-sine shock pulse |
|  | storage | 50 g, 11 ms duration, half-sine shock pulse |
| Electrical |  |  |
| Power | maximum | 8 W |
|  | typical | 3.5 W |
| Current | maximum | 5V @ 1.6A |
|  | typical | 5V @ 0.7A |
| Other |  |  |
| Weight |  | 3.5 oz (105 g) |
| Addresses |  |  |
| Memory |  | A000h - C7FFh |
| I/O |  | 3B4h - 3DEh |

Table 1. EXM-13A Environmental and Electrical Specifications.