# 2.  Installation

Before installing the EXM-13A, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the 
exm-13a, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-13A into an EXM carrier, such as the EMC, EXP-MC or the integrated EXM carriers in the EPC-4 or EPC-5 is straightforward.  Remove any blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-13A into the cardguides.  Firmly press the EXM-13A front panel to ensure that it has mated with the rear connector, and secure it with the thumbscrews.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

## Configuring the BIOS Setup

The EXM configuration data in the EPC to which the EXM-13A is connected needs to be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function. This is done by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu will be displayed specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen looks something like the following:

EXM Setup Screen

 		ID	OB1	OB2       
 Slot 	0	ED	01	00
      	1	FF	00	00
      	2	FF	00	00
      	3	FF	00	00
      	4	FF	00	00
      	5	FF	00	00

This displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which slot the EXM occupies.  Dotted lines indicate EXM slots that may not exist on all chassis.

EXM Slots

µ §

Figure 1. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

Note that all slots are listed even if the system configuration does not have all the  possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-13A should be set to ED.  For upward compatibility, this is the same ID that is used on the EXM-13.

OB1 should be set to 01 to enable the card.  OB1 of 00 disables the card.

OB2 is not used by the EXM-13A and should be set to 00.

## Default Initialization of Video EXMs

For a variety of reasons, the BIOS in EPCs automatically enables a video EXM if it finds that none are enabled.  The primary reason is the chicken-and-egg problem of configuring the EXM the first time; one needs to be able to see the video output of the setup program in order to configure the video EXM.

If the BIOS finds no enabled video EXM, it searches for the first video EXM by slot and enables it.  If this occurs, use the setup program to configure the EXM properly to avoid receiving an exm configuration error message every time the system is booted.

## Connecting a Monitor

A monitorxe "monitor"§ is connected via the 15-pin D connector on the EXM-13A front panel.  The EXM-13A, being a VGA graphics controller, produces analog color signals, meaning that old-style EGAxe "EGA"§, CGAxe "CGA"§, and monochrome monitors cannot be used.  VGA monitors may be color or gray scalexe "gray-scale"§; the EXM-13A automatically senses the difference between the two.  If using a multiscan monitorxe "multiscan monitor"§, make sure to set the monitor's switch to analog (not TTL).

The monitor type cannot be detected correctly if the monitor is disconnected or powered off when the EPC is powered on or reset.  Typical symptoms are lack of color on a color monitor and an error message about lack of a suitable video adapter when invoking Windows.  If a color monitor is not detected correctly under MS-DOS, the command

mode co80

can be used to switch the controller into color mode.

To ensure a reliable connection, the monitor's cable should be screwed into the EXM-13A's connector.

Connecting a monitor is not absolutely necessary; its presence or absence does not affect the operation of the system.

## Installing Super VGA Drivers

The EXM-13A supports certain video modes beyond standard VGA mode (see Appendix A for a complete list of supported modes under Windows)xe "VGA modes"§. Use of these modes requires a Super VGA or multiscan monitorxe "multiscan monitor"§, as well as special software drivers.  Drivers are provided on accompanying diskettes for Microsoft Windows, OS/2, SCO Unix and several other software packages.  Updated video drivers are available on the NCR Bulletin Board System (BBS) at (719) 596-1649.

Other operating systems may or may not have Super VGA driver support for the NCR 77C22E+ chip.  When using an operating system other than DOS, OS/2, or SCO Unix consult the operating system manual for details on supported video modes and drivers.

### Installing the Windows Drivers

The video drivers supplied on the accompanying diskettes are in compressed form.  Insert Disk #1 into the floppy drive.  Make that drive A: is the active drive.  Then type INSTALL.  This expands the files and copies them to the local hard disk.  Follow the instructions on the screen carefully.  When prompted, highlight both SOFTMODE & UTILITIES and the appropriate Windows version drivers.  Softmode is a device driver that works with the Windows driver and is required for Windows to display high resolution graphics.

After running the INSTALL program, change to the sub-directory containing SOFTMODE & UTILITIES and invoke SOFTSET.  This program tests the system and monitor for compatible video modes.  Not all Super VGA monitors support all modes.  SOFTSET then adds a line to the CONFIG.SYS file listing the supported modes; something similar to the following:

DEVICE=........\SOFTMODE.SYS  /m:2,15,16,3,......

Reboot the system so SOFTMODE is loaded.

To install the actual Windows driver, run the Windows setup program from the DOS prompt.  Change to the Windows sub-directory and, at the DOS prompt, type SETUP.  The executable will display a list of hardware recognized by Windows.  Use the cursor arrows to highlight the video display line and press ENTER.  This will display a list of supported video drivers.  At the end of the list is an entry something like "OTHER (requires hardware vendor diskette)".  The setup program will ask for a driver diskette.  Enter the name of the drive and sub-directory where the Windows drivers were copied.

VSHIFT

The VSHIFT program is used to center the screen when using the EXM-13A with the EXM-14.  To install the VSHIFT software from the Miscellaneous disk, follow the instructions below.

1.	Insert the EXM13A Miscellaneous disk into drive A:.

2.	Install the VSHIFT files onto the hard drive by running VINST.  Include the location where you want the files to be copied to. For example, to install VSHIFT in the C:\TEMP directory, type the following:

A:\VINST C:\TEMP

To use VSHIFT, type VSHIFT from the command line. You must supply an instruction following the command:  i.e., (U)p, (D)own, (R)ight, or (L)eft. For example, type VSHIFT 2L to shift the screen two vertical scan lines to the left. You can combine instructions, such as VSHIFT 2R 2D to shift the screen image two lines right and two lines down, or VSHIFT 2U 2R to shift the screen two lines up and two lines right. Whenever you enter a command, keep track of what you enter so that you will eventually discover what the best single starting instruction is.

Once you have determined the optimum VSHIFT command, insert that single instruction into your STARTUP.COM statement. Every time the machine is turned off, the monitor defaults back to the original setting.

### Installing the OS/2 Drivers

To install the OS/2 drivers, OS/2 must be loaded using the FAT file structure.  This procedure does not work with the HPFS file structure.

The following procedures are for OS/2 up to version 1.3. Refer to the following section for instructions that pertain to OS/2 version 2.1.

OS/2 Version 1.3

First, the drivers must be installed to the hard disk under DOS, either using a DOS window or by booting from a DOS floppy disk.  Run the INSTALL program on the software drivers diskette.  When prompted, highlight the OS/2 drivers for installation.  By default, the drivers are installed on the hard disk under the PM directory.

If OS/2 is running, perform a SHUTDOWN.  OS/2 locks the video driver files when running, so the system must be rebooted under DOS.  At the DOS prompt, copy all the .DLL files in the PM directory to the \OS2\DLL directory.  Change directories to \OS2\DLL.

Rename DISPLAY.DLL to DISPLAY.OLD. Rename BVHVGA.DLL to BVHVGA.OLD.

Copy BVH77C22.DLL to BVHVGA.DLL.

NCR800.DLL is the 800 x 600 driver.  NCR1024.DLL is the 1024 x 768 driver.  Copy the appropriate driver to DISPLAY.DLL.

Reboot the system to OS/2.

OS/2 Version 2.1

The OS/2 video drivers provide for higher screen resolutions, such as 1024 x 768 x 256 colors. To install the OS/2 version 2.1 video drivers, follow the instructions below:

1.	Insert the EXM13A Miscellaneous disk into drive A:

2.	Run the install program on the disk.  Don't forget to put the drive letter of your OS/2 installation.  For example, if OS/2 was installed on drive D: the following command would be used:

A:\INSTALL D

3.	Click on "Primary Display", then click on "OK".

4.	Select "NCR 77C22E+/77C32BLT SVGA Chipset."  Press "OK".

5.	Answer the prompt to indicate what type of monitor is attached.

6.	Select the appropriate driver resolution from the list and press "OK".

7.	The next panel allows you to change the source directory. Simply press "INSTALL..." and accept the default of "A:\".

8.	Files will be copied to your hard drive.  CONFIG.SYS, OS2.INI, and SYSTEM.INI will be modified to use the new SVGA drivers.

9.	A message window will be displayed stating that the system must be shutdown and rebooted for the changes to take effect. Press "OK" to continue.

If the standard VGA (640x480x16) drivers are installed after installing NCR high resolution drivers, an extra line may be left in CONFIG.SYS that will generate a warning message. This is the result of an attempt to load two virtual video drivers.     Edit CONFIG.SYS and delete the extra line.

Example: After installing NCR's SVGA drivers and reinstalling the standard VGA drivers, CONFIG.SYS may have these two lines:

DEVICE=C:\OS2\MDOS\VSVGANCR.SYS

DEVICE=C:\OS2\MDOS\VVGA.SYS

Delete (or remark) the NCR driver to use the standard VGA driver.

REM DEVICE=C:\OS2\MDOS\VSVGANCR.SYS

DEVICE=C:\OS2\MDOS\VVGA.SYS

### Installing the SCO ODT Driver

To install the SCO Unix driver, run the INSTALL program on a DOS machine.  When prompted, highlight the SCO Unix driver to be installed.  After installation, copy the driver to a floppy disk.

Boot SCO ODT as normal.  Open a shell window.  Type the following commands:

cd /usr/lib/grafinfo

mkdir ncr

cd ncr

doscp /def/fd0:pc3433.xgi

mkdev graphics

Select Update.  Select the NCR 3433 driver from the video card list.  Select the desired resolution and then the type of monitor that is connected.

Quit and save the configuration.

Reboot SCO ODT.

## Flat-Panel Displays

Flat-panel displays have a number of different conventions for interfacing to a VGA controller.  Most require some of the signals on the auxiliary connector.  Some connect to the analog color signals; others do not.  Some "non-color" flat panels operate best with the video controller in gray-scale mode; others have their own color-to-gray-scale conversion logic and require color mode.  The latter may require that the EXM be "tricked" into thinking that an analog color monitor is connected.

The EXM-13A detects the type of monitor in use at the time of system reset by sensing the three analog color signals on the DB-15 VGA connector.  If it senses a 75 ohm load on all three color signals, it initializes to color mode.  All other conditions initialize the controller to gray-scale mode.

When using a flat-panel display that requires color mode (e.g., it does its own conversion) and either doesn't connect to the analog signals or doesn't provide the 75-ohm load on the analog signals, it may be necessary to build a terminating connector that plugs into the DB-15 connector.  The terminating connector contains three 75-ohm resistors.  The resistors connect the red, green, and blue color signals (pins 1-3) to one of the ground pins (e.g., pin 5).

NOTES