# 4.  Connectors

The DB-15 monitor connector is defined below:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Red | 9 | (key) |
| 2 | Green | 10 | Ground |
| 3 | Blue | 11 | (not used) |
| 4 | (not used) | 12 | (not used) |
| 5 | Ground | 13 | Horizontal sync |
| 6 | Ground | 14 | Vertical sync |
| 7 | Ground | 15 | (not used) |
| 8 | Ground |  |  |

Table 3. DB-15 Pin-out.

The EXM-13A contains an auxiliary connector.  Its 36-pin connector is a shielded female 3M 10236-5212VE (or equivalent).  The mating connector is a 3M 
10136-6000EE male connector (or equivalent).

The pins on the auxiliary video connector on the EXM-13A are defined below.

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | EVIDEO | 19 | Ground |

| 2 | P7 | 20 | P0 |
| --- | --- | --- | --- |
| 3 | Ground | 21 | P1 |
| 4 | DCLK | 22 | P2 |
| 5 | EDCLK | 23 | Ground |
| 6 | P6 | 24 | P3 |
| 7 | Ground | 25 | P4 |
| 8 | -BLANK | 26 | P5 |
| 9 | HSYNC | 27 | Ground |
| 10 | VSYNC | 28 | ESYNC |
| 11 | Ground | 29 | Ground |
| 12 | reserved | 30 | Ground |
| 13 | reserved | 31 | Ground |
| 14 | reserved | 32 | Ground |
| 15 | Ground | 33 | Ground |
| 16 | RED | 34 | Ground |
| 17 | GREEN | 35 | Ground |
| 18 | BLUE | 36 | Ground |

Table 4. Auxiliary Connector Pin-out.

The signals on the auxiliary connector are described below.

-BLANK	This input/output signal is an alternate blanking signal to the digital-to-analog converter (DAC) in the EXM-13A.  When enabled (see ESYNC), a low input causes the DAC to drive its analog color outputs to 0V.  When disabled, this pin outputs the internal blanking signal.

BLUE	This analog output is the same as the blue output on the DB-15 connector.

DCLK	This input/output signal is the dot clock to the DAC.  When enabled (see EDCLK), it is an input whose rising edge causes the DAC to latch the digital video signals.  When disabled, it outputs the internal dot clock.

EDCLK	This input signal controls the source of the dot clock.  When high (or unconnected), the clock is internal.  When low, the DCLK input is enabled.  Also, when EDCLK is low, the internal dot clock is disconnected and DCLK used in its place.  While EDCLK is high or unconnected, the VGA miscellaneous output register must be configured so that it is not selected clock source 2.

EVIDEO	This input signal controls the source of the digital video information to the DAC.  When high (or unconnected), the digital signals are obtained internally.  When low, the digital signals are sourced from P0-P7 on the connector.

ESYNC	This input signal controls the source of the blanking and synchronization signals.  When high (or unconnected), they are obtained internally.  When low, the inputs -BLANK, HSYNC, and VSYNC are enabled as the source.

GREEN	This analog output is the same as the green output on the DB-15 connector.

HSYNC	This input/output signal is the horizontal sync signal on the VGA monitor connector.  When enabled (see ESYNC), it is an input and drives the horizontal sync signal on the VGA monitor connector.  When disabled, it outputs the horizontal sync signal.

P0-P7	These input/output signals are the digital video inputs to the 
EXM-13A's DAC.  When enabled (see EVIDEO), they are inputs that drive the DAC.  When disabled, they output the internal digital video signals.  P0-P5 represent, respectively, blue, green, red, secondary blue, secondary green, and secondary red.

RED	This analog output is the same as the red output on the DB-15 
connector.

VSYNC	This input/output signal is the vertical sync signal on the VGA monitor connector.  When enabled (see ESYNC), it is an input and drives the vertical sync signal on the VGA monitor connector.  When disabled, it outputs the vertical sync signal.

NOTES