# 2.  Installation

Before installing the EXM-13B, unpack and inspect it for shipping damage.

	do not remove the module from its anti-static bag unless you are in a static-free environment.  the 
exm-13b, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-13B into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-13B into the card guides.  Firmly press the EXM-13B front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews. DO NOT OVERTIGHTEN.

	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

Once the EXM-13B is inserted into the carrier, turn the system on and start to boot it up. The EXM configuration data in the EPC to which the EXM-13B is connected now must be modified to recognize and enable the card and select from the available options.

Refer to your specific EPC hardware manual for more complete information about setting up an EXM mondule.

### Enabling the EXM module with Phoenix BIOS

If you are using the EPC-8, EPC-9, EPC-26A, or EPC-27, you will be using the Phoenix BIOS to configure your EXM-13B. To access the BIOS Setup Menus, press the F2 functio key during the start of the boot process. You cannot reach the BIOS from the command line.

The BIOS Main Menu has a sub-menu for EXM configuration:  the EXM menu. Press the right-arrow key until you reach the EXM menu.

Figure 1. Phoenix BIOS EXM Menu.

Refer to Figure 3 for a discussion of EXM slot numbering. Once you determine the slot number the EXM-13B occupies, enter the configuration information into the ID and option byte sections of the EXM table. Defaults for the EXM-13B are an ID of ED and OB1 set to 01, with OB2 set to 00. Continue reading this chapter for more information.

### Enabling the EXM module with Award BIOS

If you are using an EPC-4, EPC-5, EPC-6, EPC-7, EPC-21, EPC-22, EPC-23, EPC-24, or EPC-26, you will be using the Award BIOS. Invoke the BIOS setup function from the command line by pressing the CTRL-ALT-ESC keys simultaneously.

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    DC  00  00
      1    DB  C1  00
      2    ED  01  00
      3    DE  00  39
      4    F5  05  00
      5    DC  F5  00

Figure 2. EXM Setup Screen.

The EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  An example of configuration data for the EXM-13B is shown in bold in slot 2. The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-13B occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

Figure 3. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

Note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-13B should be set to EDh.  OB1 is a hexadecimal value derived by combining the following:

| Reserved (bits 7 through 1) | Card Enable (bit 0) |
| --- | --- |
| Set to 0 | 0 disable 1 enable |

Figure 4. OB1 Register.

OB1 contains only the card enable bit. Thus a typical OB1 value is 01h, indicating that the card is enabled.

OB2 is not used and should be set to 00.