# 4.  Programming Interface

This chapter contains information needed to write custom software drivers for the EXM-13B.  Anyone using the EXM with software drivers other than provided with the product should read this chapter.

## Register Descriptions

If  the ~EXMID bus signal is asserted, the EXM-13B responds to accesses to I/O addresses 100h and 102h. A read from 100h returns the assigned device ID value of EDh. Writes to 100h are ignored. Only bit 0 of 102h is implemented. This is the readable and writeable card enable bit. It is cleared at any EXM interface reset, thereby totally disabling the EXM-13B until it is explicitly enabled.

The CL-GD5434 chip has a multitude of registers at various I/O locations. They correspond to IBM VGA standard I/O addresses and are far too numerous to enumerate here. Also, the CL-GD5434 responds to memory locations in the address range of A0000h through BFFFFh as well as the range of C0000h through C7FFF (BIOS). The CL-GD5434 is reset and disabled while the card enable bit is cleared and will not respond to any of the above addresses.

## Display Memory

The CL-GD5434 supports display memory in either 1 MB or 2 MB configurations.   The EXM-13B will use 256K x 16 (4 Mbit) DRAMs.  Display memory is programmable up to an MCLK rate of 50 MHz.

## Video BIOS ROM

The BIOS ROM responds to memory reads in the address range of C0000h through C7FFFh, provided that the EXM-13B is enabled.  The BIOS is in a one-time-programmable, surface-mount 256Kx8 device.

## Supported Resolutions

The EXM-13B supports the display modes shown in the following tables. Many configurations can operate with only 1 MB of display memory, but those same configurations may achieve much higher performance with 2 MB of display memory.

Windows 3.1

| Resolution | Colors | Memory (MB) | 43i Hz | 60 Hz | 70 Hz | 72 Hz | 75 Hz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 640x480 | 16 | 1 | NA | X | NA | X | X |
| 640x480 | 256 | 1 | NA | X | NA | X | X |
| 640x480 | 65536 | 1 | NA | NA | NA | NA | X |
| 640x480 | 16.8 M | 2 | NA | NA | NA | NA | X |
| 800x600 | 16 | 1 | NA | X | NA | X | X |
| 800x600 | 256 | 1 | NA | X | NA | NA | NA |
| 800x600 | 65536 | 1 | NA | X | NA | NA | NA |
| 800x600 | 16.8 M | 2 | NA | X | NA | NA | NA |
| 1024x768 | 16 | 1 | X | NA | NA | NA | NA |
| 1024x768 | 256 | 1 | X | X | NA | NA | X |
| 1024x768 | 65536 | 2 | X | NA | X | NA | X |
| 1280x1024 | 16 | 1 | X | NA | NA | NA | NA |
| 1280x1024 | 256 | 2 | X | NA | NA | NA | NA |

Table 6. Windows 3.1 Resolutions.

X=supported

NA = not supported

Windows NT v3.5

| Resolution | Colors | Memory | 43i Hz | 60 Hz | 70 Hz | 72 Hz | 75 Hz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 640x480 | 16 | 1 | NA | X | NA | NA | X |
| 640x480 | 256 | 1 | NA | X | NA | NA | X |
| 640x480 | 65536 | 1 | NA | X | NA | NA | X |
| 640x480 | 16.8 M | 2 | NA | NA | NA | NA | NA |
| 800x600 | 16 | 1 | NA | NA | X | X | X |
| 800x600 | 256 | 1 | NA | X | NA | X | X |
| 800x600 | 65536 | 1 | NA | X | NA | X | X |
| 800x600 | 16.8 M | 2 | NA | NA | NA | NA | NA |
| 1024x768 | 16 | 1 | X | X | X | NA | X |
| 1024x768 | 256 | 1 | X | X | X | NA | X |
| 1024x768 | 65536 | 2 | X | X | X | X | X |
| 1280x1024 | 16 | 1 | X | X | NA | NA | NA |
| 1280x1024 | 256 | 2 | X | X | NA | NA | NA |

Table 7. Windows NT Resolutions.

X=supported

NA = not supported

OS/2 v2.1, v3.0

| Resolution | Colors | Memory | 43i Hz | 60 Hz | 70 Hz | 72 Hz | 75 Hz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 640x480 | 16 | 1 | NA | NA | NA | NA | NA |
| 640x480 | 256 | 1 | NA | X | NA | X | NA |
| 640x480 | 65536 | 1 | NA | X | NA | X | NA |
| 640x480 | 16.8 M | 2 | NA | X | NA | X | NA |
| 800x600 | 16 | 1 | NA | NA | NA | NA | NA |
| 800x600 | 256 | 1 | NA | X | NA | X | X |
| 800x600 | 65536 | 1 | NA | X | NA | X | X |
| 800x600 | 16.8 M | 2 | NA | NA | NA | NA | NA |
| 1024x768 | 16 | 1 | NA | NA | NA | NA | NA |
| 1024x768 | 256 | 1 | X | X | X | X | X |
| 1024x768 | 65536 | 2 | NA | X | NA | X | NA |
| 1280x1024 | 16 | 1 | NA | NA | NA | NA | NA |
| 1280x1024 | 256 | 2 | X | X | NA | NA | NA |

Table 8. OS/2 Resolutions.

The RAMDAC and clock generators are internal to the CL-GD5434.  They are capable of operating at up to 135 MHz.

X=supported

NA = not supported