# 1.  Product Description

This manual contains the information you need to install and use the EXM-14 Live Motion Video controller and its accompanying software diskette.

The EXM-14 is a video acquisition board that allows live video from one of several types of sources to be displayed and manipulated on a VGA graphics screen.  It also allows images to be frozen, stored, and recalled.

The EXM-14 uses the Chips and Technologies 82C9001 PC-video chip to provide control of the video data. The data is stored in 15-bit RGB format which provides 32,768 colors to be displayed.

The EXM-14 hardware contains an EXM-bus interface, the C&T chip, a 1MB frame buffer, video output and multiplexing, A-D conversion, and a digital composite decoder circuit.

The EXM-14 is used in concert with an EXM-13/EXM-13A VGA graphics controller.  It connects to the EXM-13/EXM-13A through the EXM-13/EXM-13A feature connector.

The EXM-14 cannot be used with products that do not have two EXM slots such as the EPC-6 or with products that have integrated VGA controllers such as the EPC-2 and EPC-3.

The software supplied with the EXM-14 provides device drivers and an application programming interface library to use the EXM-14 with either Microsoft Windows or with OS/2 Presentation Manager.  An application is provided that displays full-motion video in a window. Sample code is provided to show how to use the application programming interface to the EXM-14 in a Windows or Presentation Manager application.

RadiSys ships a cable with the EXM-14 that connects the EXM-14 to the EXM-13A.

## Specifications

The following table defines the power and environmental specifications of the 
EXM-14.

| Characteristic |  | Value |
| --- | --- | --- |
| Temperature | operating | 0 to 60°C ambient |
|  | storage | -40 to 85°C |
| Humidity | operating | 5 - 90% noncondensing |
|  | storage | 8 - 95% noncondensing |
| Altitude | operating | 10,000 ft (3000 m) |
|  | storage | 50,000 ft (15,000 m) |
| Vibration | operating | 0.015 inch (0.38 mm) P-P displacement with 2.5 g peak (max) acceleration over 5-2000 Hz |
|  | storage | 0.030 inch (0.76 mm) P-P displacement with 5.0 g peak (max) acceleration over 5-2000 Hz |
| Shock | operating | 30 g, 11 ms duration, half-sine shock pulse |
|  | storage | 50 g, 11 ms duration, half-sine shock pulse |
| Power | maximum | 16 W |
|  | typical | 9 W |
| Current | maximum | +5V @ 2.3A +12V @ 260mA |
|  | typical | +5V @ 1.2A +12V @ 200mA |
| Weight (EXM-14) |  | 5.2 oz (150 g) |
| Weight (cable) |  | 4.2 oz (120 g) |

Table 1. EXM-14 Environmental and Electrical Specifications.