# 3. Connectors

## Connectors

The DB-15 monitor connector is defined as the following:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Red | 9 | (key) |
| 2 | Green | 10 | Digital Ground |
| 3 | Blue | 11 | (not used) |
| 4 | (not used) | 12 | (not used) |
| 5 | Digital Ground | 13 | Horizontal sync |
| 6 | Analog Ground | 14 | Vertical sync |
| 7 | Analog Ground | 15 | (not used) |
| 8 | Analog Ground |  |  |

Table 3. DB-15 Connector Pin-out.

The EXM-14 contains a 36-pin auxiliary connector.  This connector is a shielded female AMP 174726-3.  It mates with an AMP 174340-3 male connector with 174747 shield case kit, or equivalent.

The pins on the auxiliary video connector on the EXM-14 are defined below.

Note that the EXM-13 and EXM-13A have identical pin-outs. The EXM-13A uses a blade connector, while the EXM-13 uses pins.

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | (not used) | 19 | Ground |
| 2 | P7 | 20 | P0 |
| 3 | Ground | 21 | P1 |
| 4 | DCLK | 22 | P2 |
| 5 | (not used) | 23 | Ground |
| 6 | P6 | 24 | P3 |
| 7 | Ground | 25 | P4 |
| 8 | -BLANK | 26 | P5 |
| 9 | HSYNC | 27 | Ground |
| 10 | VSYNC | 28 | (not used) |
| 11 | Ground | 29 | (not used) |
| 12 | Red (live video) | 30 | Analog Ground (for analog video in) |
| 13 | Green (live video) | 31 | Analog Ground (for analog video in) |
| 14 | Blue (live video) | 32 | Analog Ground (for analog video in) |
| 15 | COMP/SYNC | 33 | Analog Ground (for analog video in) |
| 16 | RED (VGA) | 34 | Analog Ground (for analog in from VGA) |
| 17 | GREEN (VGA) | 35 | Analog Ground (for analog in from VGA) |
| 18 | BLUE (VGA) | 36 | Analog Ground (for analog in from VGA) |

Table 4. Auxiliary Connector Pinout.

The signals on the auxiliary connectors of the EXM-14 are described below.

-BLANK	This input signal is an alternate blanking signal from the EXM-13/EXM-13A.

BLUE (VGA)	This analog input is the blue color from the EXM-13/EXM-13A.

BLUE (live video)	This analog input is the blue color from the live video RGB source.

DCLK	This input signal is the dot clock from the EXM-13/EXM-13A.

GREEN (VGA)	This analog input is the green color from the EXM-13/EXM-13A.

GREEN (live video)	This analog input is the green color from the live video RGB source.

HSYNC	This input signal is the horizontal sync signal from the EXM-13/EXM-13A.

P0-P7	These input signals are the digital video inputs from the EXM-13/EXM-13A

RED (VGA)	This analog input is the red color from the EXM-13/EXM-13A.

RED (live video)	This analog input is the RED color from the live video RGB source.

COMP/SYNC	This analog input contains the composite video input of the live video source or it contains the sync information for RGB input.

VSYNC	This input signal is the vertical sync signal from the 
EXM-13/EXM-13A.

NOTES