# Appendix A:  Video Standards

## Timing parameters summary

RS-170, M/PAL

Horizontal line frequency: 15,750 Hz (period: 63.492 uS)

Horizontal blanking interval: 10.80 uS

Time from HS to active video: 9.6 uS

Field frequency: 60 Hz (period: 16.667 mS)

Lines per frame: 525

RS-170A, M/NTSC, (US, Japan)

Horizontal line frequency: 15,734 Hz (period: 63.556 uS)

Horizontal blanking interval: 10.90 uS

Time from HS to active video: 9.4 uS

Field frequency: 59.94 Hz (period: 16.6833 mS)

Lines per frame: 525

N/PAL

Horizontal line frequency: 15,625 Hz (period: 64.00 uS)

Horizontal blanking interval: 10.88 uS

Time from HS to active video: 9.6 uS

Field frequency: 50 Hz (period: 20 mS)

Lines per frame: 625

B, G, H, I, D, K, K1, L/PAL or SECAM

Horizontal line frequency: 15,625 Hz (period: 64.00 uS)

Horizontal blanking interval: 12.00 uS

Time from HS to active video: 10.50 uS

Field frequency: 50 Hz (period: 20 mS)

Lines per frame: 625

## Level summary

RS-170 with sync

Absolute amplitude: 1.4V P-P.

Blanking level: 0 IRE (0.40 volts)

Black level: 7.5 IRE (0.475 volts)

Reference white: 100 IRE (1.40 volts)

Sync level: -40 IRE (0 volts)

RS-170 without sync

Absolute amplitude: 1.0V P-P.

Blanking level: 0 IRE (0 volts)

Black level: 7.5 IRE (0.075 volts)

Reference white: 100 IRE (1.00 volts)

RS-170A

(same as RS-170 with sync)

RS-343 with sync

Absolute amplitude: 1.00V P-P.

Blanking level: 0 IRE (0.286 volts)

Black level: 7.5 IRE (0.339 volts)

Reference white: 100 IRE (1.00 volts)

Sync level: -40 IRE (0 volts)

RS-343 without sync

Absolute amplitude: 0.714V P-P.

Blanking level: 0 IRE (0 volts)

Black level: 7.5 IRE (0.053 volts)

Reference white: 100 IRE (0.714 volts)

M/NTSC, M/PAL, N/PAL

(Typical level 1V. Absolute level not defined by spec.)

(see RS-170 with sync)

B, G, H, I, D, K, K1, L/PAL or SECAM

(Typical level 1V. Absolute level not defined by spec.)

Blanking level: 0 IRE (0.300 volts)

Black level: 0 IRE (0.300 volts)

Reference white: 100 IRE (1.00 volts)

Sync level: -43 IRE (0 volts)