# Appendix B:  Glossary

ADC or A/D - Analog-to-Digital Converter. Converts a voltage to a digital binary number.

Blanking level - "blacker than black", a level guaranteed to blank the display. (PAL and SECAM define reference black as the same as blanking level).

Chrominance or Chroma - the color information in a video signal.

Composite - Video signal where sync, color (chroma) and intensity (luma) are encoded in one waveform.

DAC - Digital-to-Analog converter. Converts a digital value to a voltage

IRE - A numerical representation of video waveforms. Unitless. For NTSC, RS170, and RS343, 100 IRE is defined as reference white, 0 blanking level. Reference black is 7.5 IRE. Sync is -40 below blanking. PAL and SECAM define these points slightly differently.

Luminance or Luma - the intensity information in a video signal.

NTSC - A composite video encoding standard used in north America and Japan. There are many variations, but the generic NTSC/M is the most common.

PAL - A composite video standard used in Europe. Again, there are several variations. PAL/B and PAL/G are the most common.

Reference black - the "blackest" black level

Reference white - the "whitest" white level

RGB - Color video that is provided on 3 separate signals for Red, green, and blue.

RS-170 - a standard for video specifying timing and levels. Commonly used for TV closed circuit and studio facilities.

RS-343 - a higher-resolution video graphics standard. Defines levels differently than RS-170. Allows variation in timing. VGA monitors use RS-343 video.

Sync - the timing information for the video waveform. May be contained on a composite video signal or separately.

Sync-on-green - RGB video format where the sync signal is carried similar to composite video on the Green video signal.

SVHS - Super VHS. A form of composite video where chroma is carried on a separate signal from the luma and sync.

SECAM - Another standard, used in France and some third-world countries. SECAM differs from PAL and NTSC in that the color difference signal is frequency-modulated instead of QAM (quadrature amplitude modulated).