# 1.  Product Description

This manual contains the information you need to install and use the EXM-16 Adaptec SCSI and Floppy Disk Drive Controller EXM.

The EXM-16 SCSI controllerXE "Adaptec:SCSI Controller"§ is based on the Adaptec AIC-6360XE "Adaptec:AIC-6260"§ SCSI controller chip. This controller is compatible with the SCSI-2XE "SCSI:SCSI-2"§ specification and CCSXE "SCSI:CCS (common command set)"§ (common command set).  This controller is supported by a large base of software drivers.  The drivers support a range of operating systems (DOS, OS/2, Novell and SCO UNIX)XE "Supported Operating Systems"§ and SCSI devices, including fixed disks, removable disks, streaming tapes, CD ROMs, and digital audio tapes.XE "Supported devices"§  A BIOS extension ROMXE "BIOS:extension ROM"§ is provided for the SCSI controller allowing a SCSI disk to be a boot device.  A high density 50-pin SCSI-2 connector is provided on the front panel.  For a complete description of the supported SCSI devices and the documentation of the SCSI driver softwareXE "Software drivers"§, please refer to the companion SCSI Controller Software GuideXE "Software:SCSI Controller Software Guide"§.

The Floppy Disk Drive controller is based on the Western Digital 37C65XE "Western Digital:Floppy Controller"§ Floppy Disk controller chip and is fully compatible with the IBM PC/AT architecture.  A 34-pin ribbon connector header is provided for connecting to an externally mounted Floppy Disk Drive.  The floppy interface supports one or two floppy drives.  Power for a 3.5" floppy disk drive may be optionally supplied through the front panel connector.  Power is fused with a socketed 2 Amp fuseXE "Fuse"§, and is selected by a jumperXE "Fuse:jumper"§ on the EXM-16 circuit board.

Option registersXE "Registers"§ are provided to enable the SCSI and floppy controllers independently, locate the SCSI BIOS extension ROM in different regions of memory, select which interrupt and DMA channel to use for the SCSI controller, and select between two alternate I/O register regions for the SCSI controller.  The floppy controller is mapped to the standard PC/AT floppy disk I/O register addresses and uses the standard PC/AT floppy disk interrupt.

## CompatibilityXE "Compatibility"§XE "Limitations:with EPCs"§XE "Limitations:with devices"§

The EXM-16 can be used with any RadiSys EPC with the following restrictions.

·	The EXM-16 is fully compatible with the EXM-MXXE "EXM-MX"§.  When using the EXM-16 and the EXM-MX, both of which have floppy controllers, either floppy controller can be enabled, but only one of the floppy controllers can be enabled at a time.

·	The EXMbus interface of the EXM-3XE "EXM-3"§, the EXM-9XE "EXM-9"§, the EXP-MXXE "EXP-MX"§ and the EXP-MSXE "EXP-MS"§ preclude their use together with an EXM-16 if the floppy controller in the EXM-16 is enabled.

·	The EXM-16 cannot be used in the EXM slot of an EPC-2e.  The EPC-2e EXM slot only supports 8-bit EXMs.  The EXM-16 requires 16-bit transfers.

·	The EXM-16 always has the SCSI terminationXE "SCSI:termination"§ installed and so it can only be used as an end device on the SCSI chain.

The EPC setup menu for the fixed disk drive does not relate to any disk drives attached to the EXM-16.  In particular, do not specify SCSI for the fixed disk drive in the EPC setup screen unless you are using the EXM-16 with an EXP-MS which contains an integrated SCSI disk and controller.  The SCSI setting in the EPC setup screen enables the SCSI BIOS for the EXP-MS SCSI controller which is not compatible with the EXM-16.  The recommended setting for the EPC fixed disk setup menu is NONE.  The EXM-16 contains its own BIOS extension ROM that can be enabled by bits in the configuration registers.  These registers are described in a later chapter.

## Specifications

The following table defines the power and environmental specificationsXE "Environmental specifications"§ of the 
EXM-16.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| TemperatureXE "Temperature"§ | operating | 0 to 60°C ambient |
|  | storage | -40 to 85°C |
| HumidityXE "Humidity"§ | operating | 5% - 95% noncondensing |
|  | storage | 5% - 95% noncondensing |
| AltitudeXE "Altitude"§ | operating | 10,000 ft (3000 m) |
|  | storage | 50,000 ft (15,000 m) |
| VibrationXE "Vibration"§ | operating | 0.015 inch (0.38 mm) P-P displacement with 2.5 g peak (max) acceleration over 5-2000 Hz |
|  | storage | 0.030 inch (0.76 mm) P-P displacement with 5.0 g peak (max) acceleration over 5-2000 Hz |
| ShockXE "Shock"§ | operating | 30 g, 11 ms duration, half-sine shock pulse |
|  | storage | 50 g, 11 ms duration, half-sine shock pulse |
| Electrical |  |  |
| PowerXE "Power"§ | maximum | 9.5 W |
|  | typical | 6.5 W |
| Current XE "Current "§ | maximum | +5V @ 1.8A |
|  | typical | +5V @ 1.3A |
| Other |  |  |
| Weight XE "Weight "§ |  | 3.5 oz |

Table 1. EXM-16 Specifications.

NOTES