# 2.  InstallationXE "Installation"§

Before installing your EXM-16, you should unpack and inspect it for shipping damage.

*	XE "ESD"§XE "Static handling"§do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-16, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM CarrierXE "EXM Carrier"§

Insertion of the EXM-16 into an EXM carrier, such as the EXP-MCXE "EXP-MC"§ or the integrated EXM carriers in the EPC is straightforward.  First ensure that the subplane has been inserted into the backplane.  If an EXP-MC is being used, insert it in the desired VMEbus slot and attach it to the chassis.  Then remove any blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-16 into the card guides.  Firmly press the EXM-16 front panel to ensure that it has mated with the rear connector, and secure it with the thumbscrews.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

## ConnectorsXE "Connectors"§

µ §The high-density 50-pin SCSI connectorXE "SCSI:connector"§ is an AMP 174726-4 or equivalent socket and is defined as the following:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | GND | 26 | -DB0 |
| 2 | GND | 27 | -DB1 |
| 3 | GND | 28 | -DB2 |
| 4 | GND | 29 | -DB3 |
| 5 | GND | 30 | -DB4 |
| 6 | GND | 31 | -DB5 |
| 7 | GND | 32 | -DB6 |
| 8 | GND | 33 | -DB7 |
| 9 | GND | 34 | -DBP |
| 10 | GND | 35 | GND |
| 11 | GND | 36 | GND |
| 12 | GND | 37 | GND |
| 13 | No connect | 38 | TERMPWR |
| 14 | GND | 39 | GND |
| 15 | GND | 40 | GND |
| 16 | GND | 41 | -ATN |
| 17 | GND | 42 | GND |
| 18 | GND | 43 | -BSY |
| 19 | GND | 44 | -ACK |
| 20 | GND | 45 | -RST |
| 21 | GND | 46 | -MSG |
| 22 | GND | 47 | -SEL |
| 23 | GND | 48 | -C/D |
| 24 | GND | 49 | -REQ |
| 25 | GND | 50 | -I/O |

Table 2. SCSI Connector Pinout.

Notes:

1.	The minus sign next to a signal indicates active low

2.	The signal names are the same as defined in the SCSI specification

3.	The contact assignments refer to the connector contact set 2 for the Shielded Alternative 1 connector as defined in the SCSI specification

The 34-pin header for the external floppy disk driveXE "Floppy:connector"§ is defined as

µ §

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | GND | 2 | -DENS |
| 3 | No connect | 4 | No connect |
| 5 | No connect | 6 | No connect |
| 7 | GND or fused VCC | 8 | -IDX |
| 9 | GND or fused VCC | 10 | -MO2 |
| 11 | GND or fused VCC | 12 | -DS1 |
| 13 | GND | 14 | -DS2 |
| 15 | GND | 16 | -MO1 |
| 17 | GND | 18 | -DIR |
| 19 | GND | 20 | -STEP |
| 21 | GND | 22 | -WDATA |
| 23 | GND | 24 | -WGATE |
| 25 | GND | 26 | -TRK00 |
| 27 | GND | 28 | -WPROT |
| 29 | GND | 30 | -RDATA |
| 31 | GND | 32 | -SIDE |
| 33 | GND | 34 | -DSKCHNG |

Table 3. Floppy Connector Pinout.

Notes:

1.	The minus sign next to a signal indicates active low

## Adding Floppy Disk Drives

Attaching an external floppy drive to the EXM-16 requires removing a machined metal part from the front panel.  This part is fastened with two screws.  The floppy connector on the EXM-16 is a 3M series 3594 (or equivalent) header with 0.100" x 0.100" spacing and mates with a 3M series 3414 or equivalent wiremount socket.

The EXM-16 is configured at the factory for use with one or two standard 5.25" and 3.5" floppy disk drives.  These can be connected with a simple ribbon cable to the unit.  There are several configuration options available.

Floppy disk drives are addressable.  Normally, all PC disk drives are configured as the second drive of four.  A single drive connected with a straight cable will be addressed as the A: drive.  The second drive (B:), also configured as the second drive, is typically connected with a twisted cable (pins 10-16 are swapped) that swaps the drive address lines.

Some floppy drives can be powered through the data cable.  This provides a convenience to the user so that an external power supply is not needed.  These drives connect VCC through pins 7, 9, and 11 of the cable.  Other drives connect GND to these pins.  The EXM-16 supports both types of floppy drives.  However, care must be taken when connecting the EXM-16 to the floppy so that VCC and GND are not shorted together.  In particular, be careful with floppy drives that provide jumpers to configure whether VCC or GND is connected to these signals.  If you are unsure which type of drive you have or don't know how the jumpers are configured for a specific drive, determine this from the drive specifications before connecting the drive to the EXM-16.

µ §

Figure 1.  Fuse and Jumper Location.

A jumperXE "Floppy:jumper"§ on the EXM-16 selects either GND or fused VCC to pins 7, 9, and 11 of the floppy connector.  This is a two position jumper (three stake pins) that is labeled "Enable +5 -> FDD" with positions that are labeled ON or OFF.  Select the ON position to connect VCC to pins 7, 9, and 11 of the floppy connector.  Select the OFF position to connect GND to pins 7, 9 and 11 of the floppy connector (see figure above).  If the jumper is not installed, pins 7, 9, and 11 of the floppy connector are shorted together but unconnected to anything else.

If the jumper is set to the ON position and the floppy requires GND on these pins, then upon applying power to the EXM-16, the fuse will be blown because of a short circuit from VCC to GND.

If the jumper is set to the ON position, then using a twisted cable will blow the fuse.  This configuration is not supported.

The following table summarizes the possible configurations:

| EXM-16 Jumper position | Floppy power source | Cable type | Drive |
| --- | --- | --- | --- |
| Not installed | External | Straight | A: |
| Not installed | External | Twisted | B: |
| OFF | External | Straight | A: |
| OFF | External | Twisted | B: |
| ON | Data cable | Straight | A: |

Table 4. EXM-16 Configurations.

The part number for the fuse XE "Fuse "§is Schurter 3402.0012.22.  The factory setting for this jumper is neither the ON or OFF position.  This allows you to connect to any floppy disk regardless of its jumper positions without causing damage to the fuse.  Be sure that when using either the ON or OFF position of the jumper, that the floppy drive is configured to match the voltage applied to pins 7, 9, and 11 of the floppy connector.