# 4.  Programming Interface

## RegistersXE "Configuration Registers"§

| Register | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID | 1 | 1 | 1 | 1 | 1 |  | 1 |  | 0 | 1 |  | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Option Byte 1 | DMA channel |  | Interrupt |  | IO base |  | SCSI enable |  | Floppy enable | Card enable |  | 102 |
| Option Byte 2 | unused |  |  |  |  | Dual Speed |  | ROM base address |  |  | 103 |  |

EXM Configuration RegistersXE "Registers:Configuration"§

The EXM Configuration Registers are standard EXM registers for device identification and configuration.  The EXM-16 responds to accesses to these ports only if EXMbus line -EXMID is asserted.  An 8-bit read from I/O address 100h returns the value DF, the device IDXE "Device ID"§ of the EXM-16.  A read/write configuration register appears at I/O address 102h and 103h.  The defined bits are:

Card enableXE "Card enable"§ specifies whether the EXM-16 is enabled (1) or disabled (0).  If disabled, the EXM-16 will not respond to and floppy or SCSI I/O or memory addresses; it will not assert interrupts or DMA requests nor respond to DMA cycles; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h and 103h, and then only if EXMbus line -EXMID is asserted.

Floppy enableXE "Floppy:enable"§ specifies whether the floppy diskette controller is enabled (0) or disabled (1).  If Card enable is set and floppy enable is clear, the I/O addresses 3F0-3F7 and the floppy diskette controller are active.  Otherwise, the floppy controller is disabled.  In the disabled state, it will not respond to accesses to I/O addresses 3F0-3F7 or DMA cycles and IRQ6 and DRQ2 are tristate.  Note that if this bit is 0, other EXMs or DISK modules that contain enabled floppy controllers may conflict causing the system to fail.  Be sure that there is only one floppy controller enabled in the system at a given time.

SCSI enableXE "SCSI:enable"§ specifies whether the SCSI controller is enabled (1) or disabled (0).  If Card enable and SCSI enable are set, the SCSI controller is active.  When active, the SCSI controller will respond to accesses to its I/O space as defined by the I/O base address bit and it will respond to memory accesses to the BIOS extension ROM if enabled.  It will also DMA requests and respond to DMA acknowledges if enabled by the DMA channel selection bits and will generate interrupts on the selected interrupt signal as specified by the interrupt bits.  Otherwise, it is disabled.  When disabled, the SCSI controller will not respond to any I/O or memory address and will not signal interrupts or DMA requests.

I/O baseXE "I/O base"§ specifies the base address of the I/O registers to which the SCSI controller will respond.  If 0 the base address is 140h; if 1 the base address is 340h.

InterruptXE "Interrupt"§ selects the interrupt channel on which the SCSI controller will signal interrupt requests according to the following table:

00	no IRQ (interrupts are disabled)
01	IRQ9(2)
10	IRQ12
11	IRQ11

DMA channelXE "DMA channel"§ selects the DMA channel according to the following table:

00	no DMA (disabled)
01	reserved
10	reserved
11	channel 0 (8-bit DMA)

Dual SpeedXE "DMA channel"§ selects the appropriate crystal according to the following table:

0	Dual-speed floppy drive
1	Single-speed floppy drive

ROM base addressXE "ROM base address"§ locates the 16K BIOS extension ROM in memory according to the following table:

000	No ROM (disabled)
001	C0xxxh
010	C8xxxh
011	CCxxxh
100	D0xxxh
101	D4xxxh
110	D8xxxh
111	DCxxxh

Take care when setting the ROM base address.  Some EPCs cannot map accesses to the C-page of DOS memory.  For those systems, use one of the D-page settings.

The extension ROMXE "BIOS:extension ROM"§ must be enabled if you want to boot from a SCSI disk drive that is attached to the EXM-16.  Also, the fixed disk setting in the EPC setup screen fixed disk menu should be set to NONE.

AIC-6360 Registers

XE "AIC-6260:Registers"§XE "Registers: AIC-6260"§The following registers are offsets from the base address:

| Primary Address | Secondary Address | Read Register | Write Register |
| --- | --- | --- | --- |
| Base Address | Base Address | SCSISEQ | SCSISEQ |
| Base + 1h | Base + 1h | SXFRCTL0 | SXFRCTOL0 |
| Base + 2h | Base + 2h | SXFRCTL1 | SXFRCTL1 |
| Base + 3h | Base + 3h | SCSISIG1 | SCSISIG0 |
| Base + 4h | Base + 4h | None | SCSIRATE |
| Base + 5h | Base + 5h | SELID | SCSIID |
| Base + 6h | Base + 6h | SCSIDAT | SCSIDAT |
| Base + 7h | Base + 7h | SCSIBUS | None |
| Base + 8h | Base + 8h | STCNT0 | STCNT0 |
| Base + 9h | Base + 9h | STCNT1 | STCNT1 |
| Base + Ah | Base + Ah | STCNT2 | STCNT2 |
| Base + Bh | Base + Bh | SSTAT0 | CLRSINT0 |
| Base + Ch | Base + Ch | SSTAT1 | CLRSINT1 |
| Base + Dh | Base + Dh | SSTAT2 | None |

Table 5. AIC-6360 Registers.

| Base + Eh | Base + Eh | SSTAT3 | SCSITEST |
| --- | --- | --- | --- |
| Base + Fh | Base + Fh | STAT4 | CLRSERR |
| Base + 10h | Base + 10h | SIMODE0 | SIMODE0 |
| Base + 11h | Base + 11h | SIMODE1 | SIMODE1 |
| Base + 12h | Base + 12h | DMACNTRL0 | DMACNTRL0 |
| Base + 13h | Base + 13h | DMACNTRL1 | DMACNTRL1 |
| Base + 14h | Base + 14h | DMASTAT | None |
| Base + 15h | Base + 15h | FIFOSTAT | None |
| Base + 16h | Base + 16h | DMADATA | DMADATA |
| Base + 17h | Base + 17h | None | None |
| Base + 18h | Base + 18h | BRSTCNTRL/ DMADATA | BRSTCNTRL/ DMADATA |
| Base + 19h | Base + 19h | None/ DMADATA | None/ DMADATA |
| Base + 20h | Base + 20h | PORTA/ DMADATA | PORTA/ DMADATA |
| Base + 21h | Base + 21h | PORTB/ DMADATA | PORTB/ DMADATA |
| Base + 22h | Base + 22h | REV | None |
| Base + 23h | Base + 23h | STACK | STACK |
| Base + 24h | Base + 24h | None | Test |
| Base + 25h | Base + 25h | ID | None |

Table 5. AIC-6360 Registers, ctd.

For a complete explanation of the SCSI controller registers, refer to the documentation on the AIC-6360 controller chip.

WD37C65 RegistersXE "Registers:WD37C65

"§

| Floppy Operations Register | 3F2 |
| --- | --- |
| Floppy Command Register | 3F4 |
| Floppy Data Register | 3F5 |
| Alternate Status Digital Output Register | 3F6 |
| Floppy Control Register | 3F7 |

For a complete explanation of the floppy diskette controller registers, refer to documentation on the WD37C65 controller chip.  The floppy diskette controller in the EXM-16 uses interrupt IRQ6 and DMA channel 2.