# 1.  Product Description

This manual contains the information you will need to install and use the EXM-18 parallel and serial I/O expansion module.

The EXM-18 is an expansion module containing a single PC compatible parallel port and a PC compatible serial port.  There are two different versions of the 
EXM-18; one has an RS-232 serial port and the other has RS-422.  The serial port standard for each card is clearly labeled on the front panel to the left of the serial port.  Throughout this manual, sections that refer to both versions of the expansion module will refer generically to the EXM-18.  For those sections specific to each version, that section will refer to the particular serial standard.

The EXM-18 is fully software configurable via the setup screen allowing the user to enable or disable the card and select the I/O address and interrupt level for each port.

## Specifications

The following table defines the environmental and power specifications of the 
EXM-18.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 125° C |
| Humidity | operating | 0 - 95% (non-condensing) |
|  | storage | 0 - 95% (non-condensing) |
| Vibration | operating | .015"PP 2.5g (max) 5-2000 Hz |
|  | storage | .030"PP 5g (max) 5-2000 Hz |
| Shock | operating | 30g 11 msec duration |
|  | storage | 50g 11 msec duration |
| Electrical |  |  |
| EXM-18-422 | maximum | +5V @ 1 A |
|  | typical | +5V @ 700 mA |
| EXM-18-232 | maximum | +5V @ 600 mA |
|  | typical | +5V @ 400 mA |

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.