# 2.  Installation

Before installing your EXM-18, you should unpack and inspect it for shipping 
damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-18, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-18 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-18 into the card guides.  Firmly press the EXM-18 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

The EXM configuration data in the EPC to which the EXM-18 is connected needs to be modified to recognize and enable the card and select from the available options.  Follow the instructions in the EPC reference manual for your specific EPC model to invoke the BIOS setup function. This is typically done by pressing the CTRL-ALT-ESC keys simultaneously.  EPC system configuration procedures differ slightly depending on BIOS versions.

### Enabling the EXM module

Once in the setup program, a menu will be displayed specifying which function keys are available for further configuration.  Press the designated function key to invoke the EXM menu.  The screen looks something like

EXM Setup Screen

           ID OB1 OB2       
 Slot 0    DD  FD  00
      1    FF  00  00
      2    FF  00  00
      3    FF  00  00
      4    FF  00  00
      5    FF  00  00

This displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which slot the EXM occupies.

EXM Slots

µ §

Figure 1: EXM Slot Numbering

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

Note that all slots are listed even if the system configuration does not have all the  possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-18 should be set to DD.

OB1 is a hexadecimal value derived by combining the following:

| RTS Gate Enable (bit 7) | Parallel port I/O select (bit 6) | Parallel port base address select (bit 5) | Serial port interrupt select (bits 4 & 3) | Serial port base address (bits 2 & 1) | EXM-18 card enable (bit 0) |
| --- | --- | --- | --- | --- | --- |
| RS232 - x - don't care RS422 - 0 - disabled 1 - enabled | 0 - input 1 - output | 0 - 278, IRQ5 1 - 378, IRQ7 | 00 - none 01 - IRQ3 10 - IRQ4 11 - IRQ2 | 00 - none (disabled) 01 - 2F8 10 - 3E8 11 - 2E8 | 0 - disabled 1 - enabled |

For example, a typical OB1 value is FDh (11111101) indicating RTS gate enabled, the parallel port is set for output, the parallel port is set for a base address of 378h, IRQ7.  The serial port is set to IRQ2, 3E8 (COM3) and the card is enabled.

RTS Gate enable - This bit has no effect on the RS-232 version.  For the RS-422 version, when this bit is set (1) the 422 port is enabled for output only when RTS is asserted.  This bit should be set (1) for RS-423 multi-drop applications.  When clear (0), outputs are always enabled.

Parallel port I/O select - The parallel port is not simultaneously bi-directional.  When this bit is set (1), the port is configured for output, when clear (0) the port is set for input.

Special Note:

To allow the parallel port to be used for input, the direction bit (bit 5 of register 2) on the 16C452 must also be programmed for input.  To program this bit, read register 2 (addressed at parallel port base address + 2).  Next OR the data with 10h.  Then write the data out to the same address.

Since the parallel port defaults to output at power-up, this will have to be done after each hardware reset (power-on, front panel reset, loss of power reset, etc.).

Parallel port base address select - The standard PC architecture defines the parallel port base address and interrupt levels together.  Therefore one selection sets both.  If LPT1 already exists in the system, this bit must be clear (0) so the interrupt and base address of this port do not conflict with the existing LPT1.  When using DOS, whichever port is found first during power-up will be assigned the alias LPT1 and the second parallel port found will be assigned LPT2.  Unlike serial ports, the base address does not determine which port is assigned which alias.  At power-up, the system will check the motherboard (CPU) first, then check each EXM slot starting at slot 0.

Serial port interrupt select -  These bits determine which hardware interrupt level the serial port will use.  Keep in mind that COM1: is IRQ4  and COM2 is IRQ3.  These bits should be set to not conflict with any other port in the system.

Serial port base address - These bits determine which base address the serial port will respond to. 00 indicates that the serial port is disabled.  Like all standard PCs, serial port aliases (COM1:, COM2:, etc) are determined by the base address.  These bits should be set so that the base address of this serial port does not conflict with other serial ports in the system.  COM2 is at 2F8h, COM3 at 3E8h, and COM4 at 2E8h.

Card enable - This bit determines whether the EXM-18 will be enabled or disabled at power-up.

OB2 is not used by the EXM-18 and should be set to 00.