# 3. Operation

The EXM-18 provides both a standard 25-pin parallel port and a 9-pin serial port, either RS-232 or RS-422.  Both ports are implemented using a single VLSI VL16C452 chip.  The VL16C452 provides two serial channels, channel 0 and channel 1.  Channel 0 is not used.  The serial port on the EXM-18 is wired to channel 1.

This chip does not support HDLC and SDLC protocols.  For detailed programming information, see the VLSI Technology data manual.

The driver chips used are National Semiconductor DS3691M.

The RS422 version uses 100 ohm, line to line termination.

The input clock speed is set at 1.8432 MHz.  This provides baud rates based on the following divisors:

| Desired Baud rate | Divisor used | Desired Baud rate | Divisor used |
| --- | --- | --- | --- |
| 50 | 2304 | 2000 | 58 |
| 75 | 1536 | 2400 | 48 |
| 110 | 1047 | 3600 | 32 |
| 134.5 | 857 | 4800 | 24 |
| 150 | 768 | 7200 | 16 |
| 300 | 384 | 9600 | 12 |
| 600 | 192 | 19200 | 6 |
| 1200 | 96 | 38400 | 3 |
| 1800 | 64 | 56000 | 2 |

### Connectors

The parallel port is a standard DB-25 with the pin-out defined as:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Strobe | 14 | Auto line feed |
| 2 | DB0 | 15 | Error |
| 3 | DB1 | 16 | Initialize printer |
| 4 | DB2 | 17 | Select in |
| 5 | DB3 | 18 | Signal ground |
| 6 | DB4 | 19 | Signal ground |
| 7 | DB5 | 20 | Signal ground |
| 8 | DB6 | 21 | Signal ground |
| 9 | DB7 | 22 | Signal ground |
| 10 | Acknowledge | 23 | Signal ground |
| 11 | Busy | 24 | Signal ground |
| 12 | Paper end | 25 | Signal ground |
| 13 | Select |  |  |

The Serial port is a Micro-D 9-pin female connector.  The mating connector is an ITT Cannon MDSM-9SC-Z11 male connector or equivalent.  The pin-out for each version of the serial port is shown below.

RS-232 version:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | CD | 6 | DSR |
| 2 | RxD | 7 | RTS |
| 3 | TxD | 8 | CTS |
| 4 | DTR | 9 | RI |
| 5 | Gnd |  |  |

RS-422 version:

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Gnd | 6 | CTS - |
| 2 | RTS+ | 7 | CTS+ |
| 3 | RTS- | 8 | RxD+ |
| 4 | TxD+ | 9 | RxD- |
| 5 | TxD- |  |  |