# 1.  Product Description

This manual contains the information required to install and use the EXM-19 digital I/O expansion module.

The EXM-19 is available in three versions:

1. The EXM-19-24, which contains 24 I/O channels

2. The EXM-19-48, which contains 48 I/O channels

3. The EXM-19-OPTO, which has 24 opto-isolated input/output

connections.

Within this manual, any section referring generically to the EXM-19 applies to all versions.  Those sections specific to each version indicate which version is being addressed.

The EXM-19-24 is compatible with the Keithley Metrabyte PIO-12 and the Computer Boards, Inc. CIO-DIO24 board.

The EXM-19-48 is compatible with the Computer Boards, Inc. CIO-DIO48 board.

The EXM-19-OPTO is identical to the EXM-19-24 but provides optical isolation for all inputs and outputs, using Sharp PC314A opto-isolators.

On the bottom of the front panel of the EXM-19-OPTO are the words "Optoisolated Digital IO." There are several versions of the EXM-19-OPTO, and the exact combination of inputs and outputs depends on build-time options specifically ordered. The configuration of the inputs and outputs is recorded on a sticker with six hex digits, located on the circuit board just behind the front panel. Refer to Appendix A.

The EXM-19 uses one or two 82C55 controller chips.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-19.

| Characteristic |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Environmental |  |  |  |  |
| Temperature |  | operating |  | 0° to 60° C (* see below) |
|  |  | storage |  | -40° to 85° C |
| Humidity |  | operating |  | 5% - 95% (non-condensing) |
|  |  | storage |  | 5% - 95% (non-condensing) |
| Vibration |  | operating |  | .015"PP 2.5g (max) 5 to 2000 Hz |
|  |  | storage |  | .030"PP 5.0g (max) 5 to 2000 Hz |
| Shock |  | operating |  | 30g 11 msec duration |
|  |  | storage |  | 50g 11 msec duration |
| Power Consumption (EXM card only) |  |  |  |  |
| 24 line |  | maximum |  | 5V @ 0.64A |
|  |  | typical |  | 5V @ 0.40A |
| 48 line |  | maximum |  | 5V @ 1.05A |
|  |  | typical |  | 5V @ 0.70A |
| Input and Output Logic Levels (not optically isolated - see Table 2 for -OPTO) |  |  |  |  |
| Output |  | low voltage |  | 0.4V max. @ 2.5mA |
|  |  | high voltage |  | 3.0V min. @ -2.5mA |
| Input Logic |  | minimum |  | -0.5V |
| Low |  | maximum |  | 0.8V |
| Input Logic |  | minimum |  | 2.0V |
| High |  | maximum |  | 5.0V |
| Input Load |  | minimum |  | -10µA |
| (0 < Vin < 5V) |  | maximum |  | 10µA |
| Input Current |  | low |  | max. -0.4mA |
| Interrupt Inputs |  | high |  | max. 20µA |
| Maximum current available through the front panel voltage pins |  |  |  |  |
|  | +5V |  | 2A |  |
|  | +12V ( 24 line version only ) |  | 2A |  |
|  | -12V ( 24 line version only ) |  | 1A |  |

Table 1. EXM-19 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.

The EXM-19-OPTO is similar to the EXM-19-24 in most respects. Additional specifications for the EXM-19-OPTO beyond those listed in Table 1 are listed below.

| Opto-isolated Input |  |
| --- | --- |
| LED Forward Voltage | ± 1.2V Typical |
| Series Resistance | 470W |
| " On" Current | > ± 1.5mA |
| "Off" Current | < ± 40µA |
| Maximum Current | ± 40 mA |
| Opto-isolated Output |  |
| Maximum Opto-isolator Power Dissipation Rating | 150mW (calculated as voltage multiplied by the amount of current the transistor has to sink) |
| "On" Voltage @ 1mA | 0.4V maximum |
| "Off" Current | 1.0µA maximum |
| "Off" Voltage maximum | +35V/-6V |

Table 2. EXM-19-OPTO Input and Output Logic Levels.

For more information about the Sharp opto-isolator used in the EXM-19-OPTO, refer to the Sharp Corp. documentation.

NOTES