# 2.  Installation

Before installing the EXM-19, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-19, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious; it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

The shaded area is the opening for the second ribbon cable.The 48-line EXM-19 provides two 50-pin standard headers (100 mil centers), one on the front panel and one on the component side of the circuit board.  The ribbon cable for the second header must be installed before inserting the EXM-19-48 into the carrier.  The front panel has a cut-out for the ribbon cable for the second header but it is not large enough for the ribbon cable connector.  See Figure 1 below.

#### Figure 1.  EXM-19-48 Front Panel.

To install this ribbon cable, the front panel must be removed so the ribbon cable connector can be inserted through the front panel opening.  Working in an ESD protected area, lay the EXM-19-48 on a flat work surface with the solder side down and the front panel to the right as shown below.

Remove these 2 screws

#### Figure 2. EXM-19-48 Orientation.

Remove the two screws indicated above and slide the front panel to the right away from the circuit board maintaining the same orientation.

#### Figure 3.  EXM-19-48 Front Panel Removal.

Insert the ribbon cable through the opening in the front panel and connect it to the H2 header shown above.  Then slide the front panel back onto the circuit board making sure that the ribbon cable lays flat against the shroud of the H1 connector.

Replace the two screws securing the front panel to the circuit board.

## Insertion in an EXM Carrier

Insertion of the EXM-19 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-19 into the card guides.  Firmly press the EXM-19 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

The EXM configuration data in the EPC to which the EXM-19 is connected needs to be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu will be displayed specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles Figure 4 below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    DB  C1  00
      2    F5  05  00
      3    DE  00  39
      4    ED  01  00
      5    DC  F5  91

Figure 4. EXM Setup Screen.

The setup screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2 for each installed EXM.

Note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present. A typical value for the EXM-19 is shown in bold letters in slot 5.

Slot, OB1 and OB2 are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-19 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 5. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-19 should be set to DCh.

OB1 is a hexadecimal value derived by combining the following:

| Unused (bits 7 - 4) | Interrupt Select (bits 3 - 1) | EXM-19 card enable (bit 0) |
| --- | --- | --- |
| 1111 | 000 - none 001 - none 010 - IRQ9 (2) 011 - IRQ3 (usually COM2) 100 - IRQ4 (usually COM1) 101 - IRQ5 (usually LPT2) 110 - IRQ6 (usually floppy disk controller) 111 - IRQ7 (usually LPT1) | 0 - disabled 1 - enabled |

Figure 6. OB1 Register.

For example, a typical OB1 value is F5h (11110101) indicating the card is using IRQ9 and the card is enabled.

OB2 is used by the EXM-19 to set the base address of the 82C55 chip(s).  The 
EXM-19-24 uses four (4) consecutive I/O addresses and the EXM-19-48 uses eight (8) consecutive I/O addresses.  See Port Addresses, page 11 for details on port addressing.

The base address must be in the range 100h-3FCh and must be on a 4-byte boundary for the 24-line EXM or on an 8-byte boundry for the 48-line EXM.

The base address is the sum of the bit values for each set bit as shown below.

| bit 7 | bit 6 | bit 5 | bit 4 | bit 3 | bit 2 | bit 1 | bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 200h | 100h | 80h | 40h | 20h | 10h | 8h | 4h |

For example, if OB2 is set to 91h (10010001), the base address is set to the following:

200h + 40h + 4h = 244h.

Make sure that the base address chosen for this EXM-19 does not conflict with any other modules that use a base I/O address (or range).

Some other common address ranges to avoid are:

1F0-1FF		IDE disk controller

278-27F		Parallel port (LPT2)

2E8-2EF	Serial port (COM4)

2F8-2FF		Serial port (COM2)

378-37F		Parallel port (LPT1)

3E8-3EF	Serial port (COM3)

3F0-3F7		Floppy disk controller

3F8-3FF		Serial port (COM1)