# 3. Operation

The EXM-19 is available in a 24-line or 48-line version, and an OPTO version.

The 24-line and 48-line versions are implemented using one or two Intel N82C55A chip(s) respectively.  Each 8255 provides three ports.  Ports A and B are 8-bit ports user-programmable for input or output.  Port C can be configured as one 8-bit port or two 4-bit input, output or bi-directional port(s).  Port C can also be used for handshaking signals for Ports A and B.

The EXM-19-OPTO is similar to the EXM-19-24, but inputs and outputs are optically isolated. Each port bit is configured at the factory to be either an input or an output, and may not be changed under program control. Refer to Appendix A.

The 8255 chip supports three different modes.  In Mode 0 (Basic Input/Output) the ports that are configured for output hold the output data written to them and can be read back.  Mode 1 is strobed I/O and Mode 2 is bi-directional strobed I/O.

## Port Addresses

The port address for each port on the 8255(s) is calculated as an offset from the base address selected by the OB2 value in the BIOS setup.

| EXM-19-24/EXM-19-OPTO |  |  | EXM-19-48 |  |  |
| --- | --- | --- | --- | --- | --- |
| Base | +0 | Port A | Base | +0 | Port AA |
|  | +1 | Port B |  | +1 | Port AB |
|  | +2 | Port C |  | +2 | Port AC |
|  | +3 | Control Register |  | +3 | A Control Register |
|  |  |  |  | +4 | Port BA |
|  |  |  |  | +5 | Port BB |
|  |  |  |  | +6 | Port BC |
|  |  |  |  | +7 | B Control Register |

Table 3. Port Addresses.

On the 48-line version, ports AA, AB, and AC are driven by the first 8255 and ports BA, BB, and BC are driven by the second 8255.  See the following diagrams for pin locations for each port.

## Connectors

The EXM-19-24 contains a single DB-37 male connector that is an AMP 748898-3 connector or equivalent.  The mating connector is an AMP 748894-3 female connector or equivalent. The pin-out for this connector is shown below.

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | INT INPT | 20 | Fused +5V |
| 2 | INT EN | 21 | GND |
| 3 | AB7 | 22 | AC7 |
| 4 | AB6 | 23 | AC6 |
| 5 | AB5 | 24 | AC5 |
| 6 | AB4 | 25 | AC4 |
| 7 | AB3 | 26 | AC3 |
| 8 | AB2 | 27 | AC2 |
| 9 | AB1 | 28 | AC1 |
| 10 | AB0 | 29 | AC0 |
| 11 | GND | 30 | AA7 |
| 12 | Not used | 31 | AA6 |
| 13 | GND | 32 | AA5 |
| 14 | Fused -12V | 33 | AA4 |
| 15 | GND | 34 | AA3 |
| 16 | Fused +12V | 35 | AA2 |
| 17 | GND | 36 | AA1 |
| 18 | Fused +5V | 37 | AA0 |
| 19 | GND |  |  |

Table 4. EXM-19-24 DB-37 Pin-out.

The pin-out for the EXM-19-OPTO is slightly different. Refer to the table below.

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | INT IN B | 20 | INT IN A |
| 2 | INT EN B | 21 | INT EN A |
| 3 | AB7 | 22 | AC7 |
| 4 | AB6 | 23 | AC6 |
| 5 | AB5 | 24 | AC5 |
| 6 | AB4 | 25 | AC4 |
| 7 | AB3 | 26 | AC3 |
| 8 | AB2 | 27 | AC2 |
| 9 | AB1 | 28 | AC1 |
| 10 | AB0 | 29 | AC0 |
| 11 | C COM | 30 | AA7 |
| 12 | Not used | 31 | AA6 |
| 13 | B COM | 32 | AA5 |
| 14 | Not used | 33 | AA4 |
| 15 | A COM | 34 | AA3 |
| 16 | Not used | 35 | AA2 |
| 17 | GND | 36 | AA1 |
| 18 | Fused +5V | 37 | AA0 |
| 19 | GND |  |  |

Table 5. EXM-19-OPTO DB-37 Pin-out.

The EXM-19-48 contains two standard 50-pin (25 x 2, 100 mil centers) male connectors labeled H1 and H2.  Both of these connectors are 3M 2550-5002-UB or equivalent. The mating connector is a 3M 3425-7000 female connector or equivalent.  The pin-outs for these connectors are shown below.

| H1 |  |  |  |
| --- | --- | --- | --- |
| Pin | Signal | Pin | Signal |
| 1 | AA7 | 26 | GND |
| 2 | AA6 | 27 | GND |
| 3 | AA5 | 28 | GND |
| 4 | AA4 | 29 | GND |
| 5 | AA3 | 30 | GND |
| 6 | AA2 | 31 | GND |
| 7 | AA1 | 32 | GND |
| 8 | AA0 | 33 | GND |
| 9 | AC7 | 34 | GND |
| 10 | AC6 | 35 | GND |
| 11 | AC5 | 36 | GND |
| 12 | AC4 | 37 | GND |
| 13 | AC3 | 38 | GND |
| 14 | AC2 | 39 | GND |
| 15 | AC1 | 40 | GND |
| 16 | AC0 | 41 | GND |
| 17 | AB7 | 42 | GND |
| 18 | AB6 | 43 | GND |
| 19 | AB5 | 44 | GND |
| 20 | AB4 | 45 | GND |
| 21 | AB3 | 46 | GND |
| 22 | AB2 | 47 | GND |
| 23 | AB1 | 48 | GND |
| 24 | AB0 | 49 | GND |

Table 6. H1/H2 Connector Pin-outs.

| 25 | Fused +5V | 50 | GND |
| --- | --- | --- | --- |
| H2 |  |  |  |
| Pin | Signal | Pin | Signal |
| 1 | BA7 | 26 | GND |
| 2 | BA6 | 27 | GND |
| 3 | BA5 | 28 | GND |
| 4 | BA4 | 29 | GND |
| 5 | BA3 | 30 | GND |
| 6 | BA2 | 31 | GND |
| 7 | BA1 | 32 | GND |
| 8 | BA0 | 33 | GND |
| 9 | BC7 | 34 | GND |
| 10 | BC6 | 35 | GND |
| 11 | BC5 | 36 | GND |
| 12 | BC4 | 37 | GND |
| 13 | BC3 | 38 | GND |
| 14 | BC2 | 39 | GND |
| 15 | BC1 | 40 | GND |
| 16 | BC0 | 41 | GND |
| 17 | BB7 | 42 | GND |
| 18 | BB6 | 43 | GND |
| 19 | BB5 | 44 | GND |
| 20 | BB4 | 45 | GND |
| 21 | BB3 | 46 | GND |
| 22 | BB2 | 47 | GND |
| 23 | BB1 | 48 | GND |
| 24 | BB0 | 49 | GND |
| 25 | Fused +5V | 50 | GND |

## Opto-Isolators

In Figures 7, 8, and 9, schematic drawings depict the workings of the opto-isolators used in the EXM-19-OPTO.

Figure 7 is a schematic of what an input looks like. ACOM is the common for the port, and AA0 to AA7 are input pins on the DB-37. This schematic assumes the 
A port is set to input.

Figure 7. Typical OPTO input.

Figure 8 assumes the A port is set to outputs.

Figure 8. Typical OPTO Output.

Figure 9. Opto-Isolated Interface to TTL.

Figure 9 shows how to interface an input or an output to TTL logic.

## Programming the 8255

When the EXM-19 is reset (power-up or EPC front-panel reset), the 8255 is reset with all lines set for basic input.  If this is the desired configuration, no other programming is necessary.  The port configuration will remain the same until a write to the control register changes it.  If a different mode or port configuration is required, the initialization section of the application code should set the port configuration.

The 8255 provides ports A, B, and C.  Ports A and B are 8-bit ports and port C is 2 4-bit ports; Port C upper (bits 4-7) and Port C lower (bits 0-3).  Both halves of Port C can be used together in basic I/O mode to act as a single 8-bit port.  Care should be taken when splitting Port C as every read and write to Port C carries 8 bits of data so unwanted information must be ANDed out of reads, and writes must be ORed with the current status of the other half of the port.

It is a characteristic of the 8255 that all ports are cleared by a write to the control register.  Consequently, if the application program repeatedly changes the port configuration, make provisions for restoring any data to the cleared ports.

The 8255 supports three different modes of operation.

Mode 0 	-   Basic I/O

·	Two 8-bit ports and two 4-bit ports (can be used as three 8-bit ports)

·	Any port can be input or output

·	Outputs are latched

·	Inputs are not latched

Mode 1 	-   Strobed I/O; part of Port C controls data transfer

·	Ports are configured in two groups - Group A and Group B

·	Each group contains one 8-bit data port and one 4-bit control/status port

·	The 8-bit data port can be either input or output.  Both inputs and outputs are latched

·	The 4-bit port is used for control and status of the 8-bit port

·	Interrupts supported for output only

Mode 2 	-   Bi-directional I/O; part of Port C controls data transfer

·	Only supported for Group A

·	One 8-bit, bi-directional bus port (Port A) and a 5-bit control port (Port C)

·	Both inputs and outputs are latched

·	The 5-bit control port (Port C) is used for control and status for the 8-bit, bi-directional bus port (Port A)

·	8-bit Port B and 3 bits of Port C can simultaneously be used as basic I/O

·	Interrupts supported for output only

Mode 0 (basic I/O) is the most commonly used mode.  Listed below are simple instructions for the use of Mode 0.  To use Mode 1 or Mode 2, consult the Intel data book for the 8255.

Setting Mode 0 for each port requires a simple write to the control register (base address + 3).  The tables below show the bit definitions for the mode control word.

| bit 7 | bit 6 | bit 5 | bit 4 | bit 3 | bit 2 | bit 1 | bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | bit 7 | Mode Set Flag | 1 |
| --- | --- | --- | --- |
|  | bits 6 & 5 | Mode Select (Group A) | 00 = Mode 0 |
| Group A | bit 4 | Port A | 1 = input 0=output |
|  | bit 3 | Port C Upper (bits 4-7) | 1 = input 0=output |
|  | bit 2 | Mode Select (Group B) | 0 = Mode 0 |
| Group B | bit 1 | Port B | 1 = input 0=output |
|  | bit 0 | Port C Lower (bits 0-3) | 1 = input 0=output |

Table 7. Mode Settings.

Note that setting the mode for Group A sets the mode for both Port A and Port C Upper; setting the mode for Group B sets the mode for both Port B and Port C Lower.

On the EXM-19-OPTO, certain pins are dedicated as inputs and certain pins are dedicated as outputs at the factory, according to customer specifications. Note that in order to use the inputs and outputs correctly, your software must programm the 8255 to match the correct input/output configuration of the opto-isolators.

## Using Interrupts

The internal interrupt capabilities of the 8255 are not connected to the interrupt controller chip.  Thus, generating EXM-19 interrupts requires an external approach.

### EXM-19-24

The EXM-19-24 has two front panel connections that provide interrupt capability: INT EN and INT INPT.  INT EN is an interrupt enable signal and is active low.  INT INPT is the actual interrupt input signal which can be connected to either an external interrupt source or to one of the output pins (from Port C) when using Mode 1 or Mode 2. The interrupt is signalled on the rising edge on INT INPT.

### EXM-19-48

The EXM-19-48 does not provide interrupt capabilities.

### EXM-19-OPTO

The EXM-19-OPTO provides two pairs of differential input signals for interrupts; INT EN A & INT EN B and INT INPT A & INT INPT B.  Interrupts are enabled when there is a voltage potential across the INT EN signal pair.  INT INPT A and INT INPT B are the differential pair used to actually signal the interrupt to the CPU.  INT INPT A and INT INPT B should have a voltage potential across the pair most of the time and the interrupt will be signalled when the potential is removed.

## Fuses

The EXM-19 provides +5V power to front panel pins for external use.  In addition, the EXM-19-24 also supplies +12V and -12V to front panel pins.  Each voltage is provided through a 2A fuse.  Note that the -12V is specified to only provide 1A of current.  Although this may seem like a discrepancy, the 2A fuse is provided for safety reasons while the 1A specification is due to a potential long-term reliability problem if the -12V provides more than 1A at high temperatures over an extended period.  All the fuses used on the EXM-19 are Schurter P/N OMF 2A fuses. The fuse sockets are located on the component side of the board.  Fuse locations are shown in the figures below.  Note that the EXM-19-48 and the EXM-19-OPTO do not have fuse sockets for +12V and -12V.

Figure 11. EXM-19-48 Fuse Location.

µ §Figure 10. EXM-19-24 Fuse Locations.

Figure 12. EXM-19-OPTO Fuse Location.

NOTES