# Appendix A: EXM-19-OPTO Hex Sticker

The directional opto-isolators on the EXM-19-OPTO are installed at the factory according to specific customer requirements. To identify the configuration, locate the identification sticker placed on the circuit board behind the front panel. Refer to the figure below.

Figure 13. EXM-19-OPTO Identification Sticker.

The ID sticker on the printed circuit board of the EXM-19-OPTO refers to the hexadecimal notation that identifies the opto-isolated inputs and outputs.

There are three channels available: A, B, and C. Each channel has four inputs or outputs mapped to a bit in the ID.

| Bit | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Channels | C7-C4 | C3-C0 | B7-B4 | B3-B0 | A7-A4 | A3-A0 |

For example, an EXM-19-FF00FF would be manufactured with the following setup:

FF00FF= 111111110000000011111111

A bit with a 1 is an opto-isolated output. A bit with a 0 is an opto-isolated input. Thus in this example, the board has sixteen outputs - eight on the C channel and eight on the A channel. The B channel is eight inputs.