# 2.  Installation

Before installing the EXM-21, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-21, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-21 into the card guides.  Firmly press the EXM-21 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews. Do not over-tighten the thumbscrews.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

Once the EXM-21 has been installed and the system has been booted, configure the BIOS setup. The EXM configuration data in the EPC must be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function from the command line by pressing the CTRL-ALT-ESC keys simultaneously.  EPC system configuration procedures differ slightly depending on BIOS versions.

### Enabling the EXM module

Once in the BIOS setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM setup screen.  A screen similar to the following displays:

ID OB1 OB2       
 Slot 0    FF  00  00
      1    DB  C1  00
      2    DA  93  00
      3    DE  00  39
      4    F5  05  00
      5    DC  F5  00

Figure 1. EXM Setup Screen

This displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.

The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which slot the EXM occupies.

EXM Slots

µ §

Figure 2.  EXM Slot Numbering

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-21 should be set to DA.

OB1

OB1 is a hexadecimal value derived by combining the following:

| Base I/O Address (bits 7-1) | Card Enable (bit 0) |
| --- | --- |
| Refer to Table 4. | 0 - Card Disabled 1 - Card Enabled |

Table 3. OB1 Register

Base I/O Address - Use these bits to set the value for the base I/O address of the EXM-21 programming registers. The base I/O address is a single setting large enough for a four-address range, four bytes in total length. The Base I/O address must be in the range 100h-3FCh, and is the sum of the bit values for each set bit as shown below:

| bit 7 | bit 6 | bit 5 | bit 4 | bit 3 | bit 2 | bit 1 |
| --- | --- | --- | --- | --- | --- | --- |
| 200h | 100h | 80h | 40h | 20h | 10h | 8h |

Table 4. Base Address Conversion

For example, a typical address setting for bits 7-1 might be 1001001, so that the base address equals the following:

200h + 40h + 8h = 248h.

Make sure that the base I/O address chosen for the EXM-21 does not conflict with any other modules that use a base I/O address (or range). Some common base I/O address ranges to avoid are the following:

1F0-1FF		IDE disk controller

278-27F		Parallel port (LPT2)

2E8-2EF	Serial port (COM4)

2F8-2FF		Serial port (COM2)

378-37F		Parallel port (LPT1)

3E8-3EF	Serial port (COM3)

3F0-3F7		Floppy disk controller

3F8-3FF		Serial port (COM1)

Card enable - This bit determines whether the EXM-21 is enabled or disabled at power-up. The default is 1, which enables the card.

Thus, a typical OB1 value is 93h, indicating the card is using address 248h and the card is enabled.

OB2

OB2 is not used by the EXM-21 and should be set to 00.

NOTES