# 1.  Product Description

This manual contains the information required to install and use the EXM-22  high-speed General Purpose Instrumentation Bus (GPIB) expansion module. The 
EXM-22 conforms to both the IEEE 488.1 and IEEE 488.2 GPIB standards.

The EXM-22 provides a high-performance connection from a RadiSys EPC to the GPIB instrumentation bus. The EXM-22 is based on the Texas Instruments 9914 GPIB controller chip, and with the exception of one interrupt line, is compatible with the Hewlett-Packard 82341A GPIB interface card. Refer to Chapter 3, Configuration, for more information on the interrupt line difference.

The EXM-22 is a produced for use with VXI systems only. Software drivers for the EXM-22 are available as part of the RadiSys EPConnect package.

There are two versions of the EXM-22: a standard EXM-22, and a higher-performance EXM-22+. The standard version supports GPIB read transfers at a 350KB/second rate, and GPIB write transfers up to 600KB/second. The 
EXM-22+ contains 1MB or 4MB of DRAM to buffer transfers, yielding read and write burst rates up to 1MB/second.

This manual will refer to both versions generally as the EXM-22.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-22.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 125° C |
| Humidity | operating | 0 - 95% (non-condensing) |
|  | storage | 0 - 95% (non-condensing) |
| Vibration | operating | .015"PP 2.5g (max) 5-2000 Hz |
|  | storage | .030"PP 5g (max) 5-2000 Hz |
| Shock | operating | 30g 11 msec duration |
|  | storage | 50g 11 msec duration |
| Electrical |  |  |
| Current | maximum | 5V @ 1.6A |
|  | typical | 5V @ 0.7A |

Table 1. EXM-22 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.