# 2.  Installation

Before installing the EXM-22, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-22, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-22 into the card guides.  Firmly press the EXM-22 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews.  Do not over-tighten the thumbscrews.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

The EXM configuration data in the EPC to which the EXM-22 is connected needs to be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu will be displayed specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles Figure 1 below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    DB  C1  00
      2    D9  FF  00
      3    F5  05  00
      4    ED  01  00
      5    DC  F5  00

Figure 1. EXM Setup Screen.

This displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  Typical configuration data for the EXM-22 is shown in bold in slot 2. The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-22 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

µ §

Figure 2. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-22 should be set to D9h.

OB1 is a hexadecimal value derived by combining the following:

| Unused (bits 7 - 3) | I/O Address Select (bits 2- 1) | EXM-22 card enable (bit 0) |
| --- | --- | --- |
| 11111 | 00 - 250-257, 650-657, A50-A57, E50-E57 01 - 270-277, 670-677, A70-A77, E70-E77 10 - 350-357, 750-757, B50-B57, F50-F57 11 - 370-377, 770-777, B70-B77, F70-F77 | 0 - disabled 1 - enabled |

Figure 3. OB1 Register.

For example, a typical OB1 value is FFh (11111111) indicating the card is using I/O address 370-377, 770-777, B70-B77, and F70-F77 and the card is enabled.

OB2 is not used and should be set to 00.

NOTES