3.  Configuration

The EXM-22 includes interrupt and DMA capability. It may be configured to use any one of the following IRQ lines: IRQ3, IRQ5, IRQ7, IRQ9, IRQ11, IRQ12, and IRQ15.

Refer to the EPConnect User's Guide for details on how to configure the EXM-22 IRQ line.

A detailed register map of the EXM-22 is beyond the scope of this manual.  Note that the important Configuration State (Cnfg State) register (located at I/O offset 0x10 to 0x17 from the I/O address base selected in OB1) is used to program and monitor the gate array and set the IRQ level that the EXM-22 card utilizes.  The IRQSEL bits (bits 2 - 0) select one of the following interrupt request levels:

| IRQSel 2, 1, 0 | IRQ level |
| --- | --- |
| 000 | IRQ9 |
| 001 | IRQ7 |
| 010 | IRQ5 |
| 011 | IRQ3 |
| 100 | IRQ15 |
| 101 | IRQ12 |
| 110 | IRQ11 |
| 111 | Invalid |

Table 2. IRQ Selection.

The "111" encoding supports IRQ10 on HP's 82341A GPIB card. The EXM expansion interface does not support IRQ10.

NOTES